Sub Network Access Protocol (SNAP)

────────────────────────────────────

(7)    (1)    (6)       (6)    (2)│<────(46-1500 bytes)────>│(4)

┌────────┬───┬─────────┬─────────┬───┬─────────────────────────┬───┐

802.3   │Preamble│SFD│Dest Addr│ Src Addr│Len│        Data     |  Pad  │FCS│

└────────┴───┴─────────┴─────────┴───┴─────────────────────────┴───┘

┌────────────────────────────┘                         └─┐

┌───────────┬──────────┬───────────────┬─────────────────┐

│ 802.2 Hdr │ SNAP Hdr │ Layer 3 Header│       Data      │

└───────────┴──────────┴───────────────┴─────────────────┘

┌──────────────┘           │          └─────────────────────────────────┐

│   (1)      (1)      (1)  │           (3)                    (2)       │

┌────────┬────────┬────────┬────────┬────────┬────────┬────────┬────────┐

│  DSAP  │  SSAP  │  CTRL  │       Protocol ID        │    EtherType    │

└────────┴────────┴────────┴────────┴────────┴────────┴────────┴────────┘

0xAA     0xAA     0x03

Protocol ID (Set to 0x000000)

Set to 0x0000F8 for Appletalk ARP (AARP)

EtherType (See ETH_TYPE.TXT for list)

The Sub Network Access Protocol (SNAP) header is used to provide

interoperability between Network Layer protocols and 802.X networks.  It

expands the 802.2 header to include the EtherType field that is necessary for

IP, ARP, AARP or IPX.