Address Resolution Protocol (ARP) (Ethertype 0x0806)

Reverse Address Resolution Protocol (RARP) (Ethertype 0x8035)

───────────────────────────────────────────────────────────────

(8)        (6)       (6)    (2) │<─────(46-1500 bytes)────>│(4)

┌───────────┬─────────┬─────────┬────┬──────────────────────────┬───┐

Eth II  │ Preamble  │Dest Addr│ Src Addr│Type│          Data            │FCS│

└───────────┴─────────┴─────────┴────┴──────────────────────────┴───┘

│                          │

┌────────────────────────────────────┘                          │

│0               8              16              24            31│

┌───────────────┼───────────────┼───────────────┴───────────────┐

│  Hardware Address Space       │  Protocol Address Space       │

├─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│Hardware Length│Protocol Length│          Op Code              │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘

│             Sender                       Hardware          -->

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘

-->         Address            │           Sender           -->

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘

-->  Protocol     Address      │           Target           -->

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

-->         Hardware                       Address             │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│             Target        Protocol        Address             │

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘

Hardware Address Space: Specifies the Network Type

┌──────┬────────────────────────────────────────┐

│ TYPE │ Description                            │

├──────┼────────────────────────────────────────┤

│ 0001 │ Ethernet (10Mb)                        │

│ 0002 │ Experimental Ethernet (3Mb)            │

│ 0003 │ Amateur Radio AX.25                    │

│ 0004 │ Proteon ProNET Token Ring              │

│ 0005 │ Chaos                                  │

│ 0006 │ IEEE 802.X                             │

│ 0007 │ ARCNET                                 │

│ 0008 │ Hyperchannel                           │

│ 0009 │ Lanstar                                │

│ 0010 │ Autonet Short Address                  │

│ 0011 │ LocalTalk                              │

│ 0012 │ LocalNet (IBM PCNet or SYTEK LocalNET) │

└──────┴────────────────────────────────────────┘

Protocol Address Space: Specifies the protocol type.  (Same as EtherTypes)

Hardware Length: Length, in bytes, of the hardware addresses.

Protocol Length: Length, in bytes, of the protocol addresses.

Opcode: Specifies the type of ARP/RARP packet

┌──────┬──────────────────────┐

│ TYPE │ Description          │

├──────┼──────────────────────┤

│ 0001 │ ARP Request          │

│ 0002 │ ARP Reply            │

│ 0003 │ RARP Request         │

│ 0004 │ RARP Reply           │

│ 0005 │ Dynamic RARP Request │

│ 0006 │ Dynamic RARP Reply   │

│ 0007 │ Dynamic RARP Error   │

└──────┴──────────────────────┘

Sender Hardware Address: Physical address of the sender (Source MAC address)

Sender Protocol Address: Protocol address of the sender (Source IP  address)

Target Hardware Address: Physical address of the target (Destination MAC addr)

Target Protocol Address: Protocol address of the target (Destination IP addr)