IP Bootstrap Protocol (BOOTP) Packet Format

─────────────────────────────────────────────

┌────────────┬───────────┬────────────┬───────────────────┬──────────────┐

│ MAC Header │ IP Header │ UDP Header │ bootp header/data │ MAC Trailer  │

└────────────┴───────────┴────────────┴───────────────────┴──────────────┘

│                   │

┌───────────────┘                   │

│                                   │

0                                31

┌────────┬────────┬────────┬────────┐

│   OP   │ HTYPE  │  HLEN  │  HOPS  │

├────────┴────────┴────────┴────────┤

│           TRANSACTION ID          │

├─────────────────┬─────────────────┤

│    SECONDS      │     UNUSED      │

├─────────────────┴─────────────────┤

│         CLIENT IP ADDRESS         │

├───────────────────────────────────┤

│          YOUR IP ADDRESS          │

├───────────────────────────────────┤

│         SERVER IP ADDRESS         │

├───────────────────────────────────┤

│        GATEWAY IP ADDRESS         │

├───────────────────────────────────┤

│      CLIENT HARDWARE ADDRESS      │

├ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ┤

│            (16 bytes)             │

├───────────────────────────────────┤

│         SERVER HOST NAME          │

├ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ┤

│            (64 bytes)             │

├───────────────────────────────────┤

│          BOOT FILE NAME           │

├ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ┤

│           (128 bytes)             │

├───────────────────────────────────┤

│       VENDOR SPECIFIC AREA        │

├ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ┤

│            (64 bytes)             │

└───────────────────────────────────┘

OP:     1 = Request

2 = Reply

HTYPE:     Network Hardware Type

┌──────┬────────────────────────────────────────┐

│ TYPE │ Description                            │

├──────┼────────────────────────────────────────┤

│ 0001 │ Ethernet (10Mb)                        │

│ 0002 │ Experimental Ethernet (3Mb)            │

│ 0003 │ Amateur Radio AX.25                    │

│ 0004 │ Proteon ProNET Token Ring              │

│ 0005 │ Chaos                                  │

│ 0006 │ IEEE 802.X                             │

│ 0007 │ ARCNET                                 │

│ 0008 │ Hyperchannel                           │

│ 0009 │ Lanstar                                │

│ 0010 │ Autonet Short Address                  │

│ 0011 │ LocalTalk                              │

│ 0012 │ LocalNet (IBM PCNet or SYTEK LocalNET) │

└──────┴────────────────────────────────────────┘

HOPS:     Initially set to 0 by the bootp client.  If the bootp server

passes the request to another bootp server on a different IP

network, the HOPS field is incremented.

HLEN:     Network Hardware Address Length

TRANS ID:     Used by diskless machines to match responses with

requests. Similar to a sequence number.

SECONDS:     The number of seconds sincs the client started to boot.

In the following fields, the client fills in as much information as possible

and fills the others with zeros. The bootp server that replies will fill in

the blanks.

CLIENT IP ADDRESS:   The bootp client's IP address.  If the client does not

know it's IP address, the bootp server that replies

will place the client's IP address in the YOUR IP

ADDRESS field below.  This is similar to RARP, except

that bootp uses UDP and can therefore work across

gateways.

YOUR IP ADDRESS:   See Client IP Adress field above.

SERVER IP ADDRESS:   If set to zero, any bootp server that receives the

request will reply.  If set to non-zero, only the

server specified will reply to the request.

GATEWAY IP ADDRESS:   This field is returned by the server to tell the

client the address of it's default gateway.

CLIENT HW ADDRESS:   The bootp client's hardware address.

SERVER HOST NAME:   Allows the use of a Host Name instead of an IP Address

to access the bootp server.

BOOT FILE NAME:   The name of the image file that the client wants to

download from the server.  This is usually the

operational program for the diskless device.  If the

Boot File Name is left blank, the server returns the

name of the boot file.

VENDOR SPECIFIC AREA:   Optional.  Standard format shown below.

(4)                 (1)     (1)      (variable)

┌────────┬───────┬───────┬───────┬───────┬───────┬────────────────┐

│           Magic Cookie         │ Type  │  LEN  │     Value      │

└────────┴───────┴───────┴───────┴───────┴───────┴────────────────┘

Magic Cookie:   4 bytes defining the format of the remaining items in

the Vendor Specific Area.

┌─────┬─────┬─────────────────────────────────────────────────────┐

│ Type│ Len │    Description: Contents of Value field             │

├─────┼─────┼─────────────────────────────────────────────────────┤

│  0  │  -  │        Padding: Zero, used only for padding         │

│  1  │  4  │    Subnet Mask: Mask of local IP subnet             │

│  2  │  4  │    Time of Day: Time of day in universal time       │

│  3  │  N  │       Gateways: IP Addresses of N/4 gateways        │

│  4  │  N  │    Time Server: IP Addresses of N/4 time servers    │

│  5  │  N  │  IEN116 Server: IP Addresses of N/4 IEN116 servers  │

│  6  │  N  │  Domain Server: IP Addresses of N/4 Domain servers  │

│  7  │  N  │     Log Server: IP Addresses of N/4 Log servers     │

│  8  │  N  │   Quote Server: IP Addresses of N/4 Quote servers   │

│  9  │  N  │     LPR Server: IP Addresses of N/4 LPR servers     │

│ 10  │  N  │        Impress: IP Addresses of N/4 Impress servers │

│ 11  │  N  │     RLP Server: IP Addresses of N/4 RLP servers     │

│ 12  │  N  │       Hostname: N bytes of client host name         │

│ 13  │  2  │      Boot Size: 2-byte size of boot file            │

│128- │  -  │      -- reserved for site specific use --           │

│254  │  -  │      -- reserved for site specific use --           │

└─────┴─────┴─────────────────────────────────────────────────────┘