802.1d Spanning Tree Bridge Protocol Data Unit

────────────────────────────────────────────────

┌──────────────┬──────────────┬───────────────────────┬───────────────┐

│ 802.X Header │ 802.2 Header │ information field     │  MAC Trailer  │

└──────────────┴──────────────┴───────────────────────┴───────────────┘

┌──────────────┘              │                       └───────────────┐

│              ┌──────────────┘                                       │

│ (1)  (1)  (1)│                                                      │

┌────┬────┬────┬──────────────────────────────────────────────────────┐

│DSAP│SSAP│CTRL│                 Information                          │

└────┴────┴────┴──────────────────────────────────────────────────────┘

42   42   03 │                                                      │

┌──────────────┘                                                      │

│                                                                     │

│(2) 1 1 1       (8)         (4)         (8)       (2) (2) (2) (2) (2)│

┌─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┐

│   │ │ │ │    Root ID    │ Cost  │   Bridge ID   │   │Age│Max│HTm│FDl│

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘

│  │ │ │                                          │   │   │   │   │

│  │ │ └──────── Flags                            │   │   │   │   │

│  │ └────────── BPDU Type                        │   │   │   │   │

│  └──────────── Protocol Version Number          │   │   │   │   │

└─────────────── Protocol Id                      │   │   │   │   │

│   │   │   │   │

Port ID  ────────┘   │   │   │   │

Message Age  ────────────┘   │   │   │

Max Age  ────────────────┘   │   │

Hello Time  ────────────────────┘   │

Forward Delay  ────────────────────────┘

Protocol ID:  Identifies the Spanning Tree Algorithm and Protocol.  Bridges

operating in automatic mode will always send BPDUs beginning

with a two-byte hexidecimal 0000.

Protocol Ver#:  Protocol Version number.  The current revision number is 00.

(802.1d-1990 Std.)

BPDU Type:  00 = Hello BPDU

80 = Topology Change Notification BPDU

7 6 5 4 3 2 1 0

┌─┬─┬─┬─┬─┬─┬─┬─┐

Flags:  │X│0│0│X│0│0│0│0│

└─┴─┴─┴─┴─┴─┴─┴─┘

│     │

│     └───────── Topology Acknowledgement Flag

│                  1 = Topology Change Notification was rcvd

│                  0 = No Ack necessary

│

└─────────────── Topology Change Flag

1 = New bridge detected

0 = No changes detected

Root ID:  The bridge ID of the bridge believed to be the root bridge.

New bridges advertise themselves as the root bridge.  Other

bridges on the network will ignore this if they have received

recent information from or about a bridge with the lowest

bridge ID on the network.  After it receives enough information

to realize that it will not be the root bridge, the new bridge

will change this field to include the bridge ID of the current

root bridge.

Path Cost:  The root path cost field gives the path cost from the

transmitting bridge to the root bridge.  The path cost

broadcast by the root bridge is always 0.  The path costs added

by each bridge in the network are assigned by the individual

bridges (range=1-65535)

Bridge ID:  The bridge ID of the transmitting bridge.  The bridge ID is made

from the 2 byte bridge label and the 6 byte MAC address of the

bridge's adapter with the lowest MAC address.

Port ID:  The port ID is used as a tie breaker whe two bridges on the

same network have the same span path cost.

Message Age:  Gives the time elapsed since the BPDU was sent by the root

bridge.  This value is set to 0 when it is sent from the root

bridge.  Each designated bridge updates this timer as it relays

the BPDUs received form the root bridge.

Max Age:  Gives the maximum allowed age of Hello BPDUs before they are

discarded.  (range=6-40 seconds)

Hello Time:  Gives the interval between transmission of Hello BPDUs by the

root bridge or a bridge attempting to become the root bridge.

(range=1-10 seconds)

Fwd Delay:  Allows bridges to prevent temporary loops in the network.  This

value is set by the root bridge and is used by all bridges on

the network.  This parameter forces bridges to pause for

several seconds before they  begin forwarding frames on the

network.  (range=4-30 seconds)