IPX Broadcasts and the TL/100

───────────────────────────────

When configuring the TL/100 for IPX Routing, one of the parameters that should

be  taken into consideration is the "Broadcast Filter" parameter.  The

broadcast filter has five possible settings:

┌───────────────────────────────────────┐

│ 0 - Don't Forward Broadcast Pkts      │

│ 1 - Forward NETBIOS Broadcast Pkts    │

│ 2 - Forward USER ADDED Broadcast Pkts │

│ 3 - Forward NETBIOS/USER ADDED Pkts   │

│ 4 - Forward ALL Broadcast Pkts        │

└───────────────────────────────────────┘

Don't Forward Broadcast Pkts:

The TL/100 will not forward any IPX broadcast packets.

Forward NETBIOS Broadcast Pkts:

The TL/100 will forward IPX NetBIOS broadcast packets to all IPX nets.

IPX NetBIOS broadcast packets are identified by an IPX Type = 20 (0x14)

and a Destination IPX Node = 0xFFFFFFFFFFFF.  The Source and

Destination Net numbers do not affect how or where the packets are

forwarded.  However,  the following rules apply:

If the Source Net number is 0x00000000, and the packet is

forwarded to another net, the TL/100 will replace the 0x00000000

with the correct Source IPX Net number.  This is true for both

broadcast and non-broadcast packets.  For example, if the packet

originated on network 0x00000100, the TL/100 would change the

Source IPX Net number in the packet from 0x00000000 to 0x00000100

before forwarding the packet.

The TL/100 will change the Destination Net number so that it

matches the network number of the network that the packet is being

broadcast onto.  For example, if the original IPX Destination

Net number is 0x00000200, and the TL/100 needs to forward the

broadcast packet to network 0x00000300,  it will change the

Destination IPX Net number from 0x00000200 to 0x00000300 before

forwarding the packet.  Note that the Destination Net number is

not modified in non-broadcast packets.

Forward USER ADDED Broadcast Pkts:

The TL/100 will forward User Added Broadcast packets.  User Added

broadcast packets are IPX packets with Socket numbers of 8000 or

higher that have a IPX Destination Node number of 0xFFFFFFFFFFFF.

Forward NETBIOS/USER ADDED Pkts:

The TL/100 will forward both NetBIOS and User Added broadcast packets

as described above.

Forward ALL Broadcast Pkts:

The TL/100 will forward all IPX broadcast packets -- that is, all

IPX packets with a Destination Node number of 0xFFFFFFFFFFFF,

regardless of the IPX packet type or socket numbers.

The tables below show whether the TL/100 will forward a particular type of

packet based on how the Broadcast Filter is configured.  Note: Filtering is

performed on the output side, therefore, a particular type of broadcast packet

can be selectively filtered or forwarded on a per-interface basis.

Don't Forward Broadcast Packets

─────────────────────────────────

╔═══════════════════════════╦═══════════════════════════╗

┌──────┬────────╢        IPX Source         ║      IPX Destination      ╟─────┐

│Brdcst│IPX Pckt╟────────┬────────────┬─────╫────────┬────────────┬─────╢TL100│

│Filter│  Type  ║  Net   │    Node    │Sckt ║  Net   │    Node    │Sckt ║ Fwd?│

╞══════╪════════╬════════╪════════════╪═════╬════════╪════════════╪═════╬═════╡

│  0   │ 0x14   ║00000000│    ANY     │NBIOS║00000000│FFFFFFFFFFFF│NBIOS║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  0   │ 0x14   ║  ANY   │    ANY     │NBIOS║  ANY   │FFFFFFFFFFFF│NBIOS║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  0   │not 0x14║ local  │    ANY     │RIP  ║ local  │FFFFFFFFFFFF│RIP  ║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  0   │not 0x14║ local  │    ANY     │SAP  ║ local  │FFFFFFFFFFFF│SAP  ║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  0   │not 0x14║00000000│    ANY     │8000+║00000000│FFFFFFFFFFFF│8000+║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  0   │not 0x14║ local  │    ANY     │8000+║ local  │FFFFFFFFFFFF│8000+║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  0   │not 0x14║ local  │    ANY     │8000+║ remote │FFFFFFFFFFFF│8000+║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  0   │not 0x14║  ANY   │    ANY     │other║  ANY   │FFFFFFFFFFFF│other║ no  │

└──────┴────────╨────────┴────────────┴─────╨────────┴────────────┴─────╨─────┘

Forward NETBIOS Broadcast Packets

───────────────────────────────────

╔═══════════════════════════╦═══════════════════════════╗

┌──────┬────────╢        IPX Source         ║      IPX Destination      ╟─────┐

│Brdcst│IPX Pckt╟────────┬────────────┬─────╫────────┬────────────┬─────╢TL100│

│Filter│  Type  ║  Net   │    Node    │Sckt ║  Net   │    Node    │Sckt ║ Fwd?│

╞══════╪════════╬════════╪════════════╪═════╬════════╪════════════╪═════╬═════╡

│  1   │ 0x14   ║00000000│    ANY     │NBIOS║00000000│FFFFFFFFFFFF│NBIOS║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  1   │ 0x14   ║  ANY   │    ANY     │NBIOS║  ANY   │FFFFFFFFFFFF│NBIOS║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  1   │not 0x14║ local  │    ANY     │RIP  ║ local  │FFFFFFFFFFFF│RIP  ║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  1   │not 0x14║ local  │    ANY     │SAP  ║ local  │FFFFFFFFFFFF│SAP  ║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  1   │not 0x14║00000000│    ANY     │8000+║00000000│FFFFFFFFFFFF│8000+║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  1   │not 0x14║ local  │    ANY     │8000+║ local  │FFFFFFFFFFFF│8000+║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  1   │not 0x14║ local  │    ANY     │8000+║ remote │FFFFFFFFFFFF│8000+║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  1   │not 0x14║  ANY   │    ANY     │other║  ANY   │FFFFFFFFFFFF│other║ no  │

└──────┴────────╨────────┴────────────┴─────╨────────┴────────────┴─────╨─────┘

Forward USER ADDED Broadcast Packets

──────────────────────────────────────

╔═══════════════════════════╦═══════════════════════════╗

┌──────┬────────╢        IPX Source         ║      IPX Destination      ╟─────┐

│Brdcst│IPX Pckt╟────────┬────────────┬─────╫────────┬────────────┬─────╢TL100│

│Filter│  Type  ║  Net   │    Node    │Sckt ║  Net   │    Node    │Sckt ║ Fwd?│

╞══════╪════════╬════════╪════════════╪═════╬════════╪════════════╪═════╬═════╡

│  2   │ 0x14   ║00000000│    ANY     │NBIOS║00000000│FFFFFFFFFFFF│NBIOS║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  2   │ 0x14   ║  ANY   │    ANY     │NBIOS║  ANY   │FFFFFFFFFFFF│NBIOS║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  2   │not 0x14║ local  │    ANY     │RIP  ║ local  │FFFFFFFFFFFF│RIP  ║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  2   │not 0x14║ local  │    ANY     │SAP  ║ local  │FFFFFFFFFFFF│SAP  ║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  2   │not 0x14║00000000│    ANY     │8000+║00000000│FFFFFFFFFFFF│8000+║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  2   │not 0x14║ local  │    ANY     │8000+║ local  │FFFFFFFFFFFF│8000+║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  2   │not 0x14║ local  │    ANY     │8000+║ remote │FFFFFFFFFFFF│8000+║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  2   │not 0x14║  ANY   │    ANY     │other║  ANY   │FFFFFFFFFFFF│other║ no  │

└──────┴────────╨────────┴────────────┴─────╨────────┴────────────┴─────╨─────┘

Forward NETBIOS/USER ADDED Packets

────────────────────────────────────

╔═══════════════════════════╦═══════════════════════════╗

┌──────┬────────╢        IPX Source         ║      IPX Destination      ╟─────┐

│Brdcst│IPX Pckt╟────────┬────────────┬─────╫────────┬────────────┬─────╢TL100│

│Filter│  Type  ║  Net   │    Node    │Sckt ║  Net   │    Node    │Sckt ║ Fwd?│

╞══════╪════════╬════════╪════════════╪═════╬════════╪════════════╪═════╬═════╡

│  3   │ 0x14   ║00000000│    ANY     │NBIOS║00000000│FFFFFFFFFFFF│NBIOS║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  3   │ 0x14   ║  ANY   │    ANY     │NBIOS║  ANY   │FFFFFFFFFFFF│NBIOS║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  3   │not 0x14║ local  │    ANY     │RIP  ║ local  │FFFFFFFFFFFF│RIP  ║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  3   │not 0x14║ local  │    ANY     │SAP  ║ local  │FFFFFFFFFFFF│SAP  ║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  3   │not 0x14║00000000│    ANY     │8000+║00000000│FFFFFFFFFFFF│8000+║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  3   │not 0x14║ local  │    ANY     │8000+║ local  │FFFFFFFFFFFF│8000+║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  3   │not 0x14║ local  │    ANY     │8000+║ remote │FFFFFFFFFFFF│8000+║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  3   │not 0x14║  ANY   │    ANY     │other║  ANY   │FFFFFFFFFFFF│other║ no  │

└──────┴────────╨────────┴────────────┴─────╨────────┴────────────┴─────╨─────┘

Forward ALL Broadcast Packets

───────────────────────────────

╔═══════════════════════════╦═══════════════════════════╗

┌──────┬────────╢        IPX Source         ║      IPX Destination      ╟─────┐

│Brdcst│IPX Pckt╟────────┬────────────┬─────╫────────┬────────────┬─────╢TL100│

│Filter│  Type  ║  Net   │    Node    │Sckt ║  Net   │    Node    │Sckt ║ Fwd?│

╞══════╪════════╬════════╪════════════╪═════╬════════╪════════════╪═════╬═════╡

│  4   │ 0x14   ║00000000│    ANY     │NBIOS║00000000│FFFFFFFFFFFF│NBIOS║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  4   │ 0x14   ║  ANY   │    ANY     │NBIOS║  ANY   │FFFFFFFFFFFF│NBIOS║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  4   │not 0x14║ local  │    ANY     │RIP  ║ local  │FFFFFFFFFFFF│RIP  ║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  4   │not 0x14║ local  │    ANY     │SAP  ║ local  │FFFFFFFFFFFF│SAP  ║ no  │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  4   │not 0x14║00000000│    ANY     │8000+║00000000│FFFFFFFFFFFF│8000+║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  4   │not 0x14║ local  │    ANY     │8000+║ local  │FFFFFFFFFFFF│8000+║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  4   │not 0x14║ local  │    ANY     │8000+║ remote │FFFFFFFFFFFF│8000+║ yes │

├──────┼────────╫────────┼────────────┼─────╫────────┼────────────┼─────╫─────┤

│  4   │not 0x14║  ANY   │    ANY     │other║  ANY   │FFFFFFFFFFFF│other║ yes │

└──────┴────────╨────────┴────────────┴─────╨────────┴────────────┴─────╨─────┘