IPX Diagnostic Packet Formats

───────────────────────────────

Diagnostic Request Packet (Dest Sckt=0456)

──────────────────────────────────────────

┌────────────┬─────────────┬─────────────────────────┬─────────────┐

│ MAC Header │ IPX Header  │ IPX Diagnostic Packet   │ MAC Trailer │

└────────────┴─────────────┴─────────────────────────┴─────────────┘

│             │                         │

│             │                         └─────────────┐

┌────────────┘             └─────────────────┐                     │

│                                            │                     │

│                ┌──── Dest ───┬──── Src ────┐ (2)       (6)       │

┌────┬───┬──┬────┬───┬────┬────┬───┬────┬────┬────┬────────────────┐

│Cksm│Len│TC│Type│Net│Node│0456│Net│Node│Sckt│EXCL│Exlusion Addr 0 │

└────┴───┴──┴────┴───┴────┴────┴───┴────┴────┴────┴────────────────┘

┌ ─ ─ ─ ─ ─ ─ ─ ──┬── ─ ─ ─ ─ ─ ─ ─ ─┬─ ─ ─ ─ ─ ─ ─ ─ ─ ┐

| Exclusion Addr1 | Exclusion Addr 2 | Exclusion Addr n |

└ ─ ─ ─ ─ ─ ─ ─ ──┴── ─ ─ ─ ─ ─ ─ ─ ─┴─ ─ ─ ─ ─ ─ ─ ─ ─ ┘

EXCL:  Number of addresses being excluded from this Configuration

request.  Range = 0 - 80.  Only workstations can be excluded.

Servers and routers ignore exclusions.

Exclusion Addr X:  Six byte address of each node that should not respond

to this request.

Diagnostic Reply Packet (Src Sckt=0456)

───────────────────────────────────────

┌────────────┬─────────────┬─────────────────────────┬─────────────┐

│ MAC Header │ IPX Header  │ IPX Diagnostic Packet   │ MAC Trailer │

└────────────┴─────────────┴─────────────────────────┴─────────────┘

│             │                         │

│             │                         └─────────────┐

┌────────────┘             └─────────────────┐                     │

│                                            │                     │

│                ┌──── Dest ───┬──── Src ────┐                     │

┌────┬───┬──┬────┬───┬────┬────┬───┬────┬────┬─────────────────────┐

│Cksm│Len│TC│Type│Net│Node│Sckt│Net│Node│0456│        ...          │

└────┴───┴──┴────┴───┴────┴────┴───┴────┴────┴─────────────────────┘

│                     │

┌───────────────────────┘                     │

│                                             │

│(1) (1) (2)  (1) (1)      (variable)         │

┌───┬───┬────┬───┬──┬ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ┐

│Maj│Min│Sckt│Cnt│ID│Extended Component Field |

└───┴───┴────┴───┼──┴ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ┤

|ID|Extended Component Field |

├ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─┤

|  |         ...             |

└ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─┘

│                            │

┌────────────┘                            │

│                                         │

│ (1)  (1)       (4)            (6)       │

┌────┬─────┬─────────────┬────────────────┐

│Nets│NType│ Net Address │ Node Address   │

└────┼─────┴─────────────┴────────────────┤

|NType| Net Address | Node Address   |

├ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─┤

|                ...                 |

└ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─ ─┘

Maj: Major Version Number of the diagnostic responder.

Min: Minor Version Number of the diagnostic responder.

SPX Sckt: The IPX Socket Number to which all IPX diagnostic replies may

sent.

Cnt: The component count.  Specifies how many Components can be found

in the packet.

ID: The Component ID.

┌───┬───────────────────────┬──────────┐

│ID │ Component Type        │ Field    │

├───┼───────────────────────┼──────────┤

│ 0 │ IPX/SPX               │ Simple   │

│ 1 │ Router Drivers        │ Simple   │

│ 2 │ LAN Drivers           │ Simple   │

│ 3 │ Shells                │ Simple   │

│ 4 │ VAPs                  │ Simple   │

│ 5 │ Router                │ Extended │

│ 6 │ File Server/Router    │ Extended │

│ 7 │ Non-dedicated IPX/SPX │ Extended │

└───┴───────────────────────┴──────────┘

Extended Component Field:

Components with ID 5-7 have an Extended Component Field immediately

following them with the following fields:

Nets: The number of local networks (i.e. directly attached networks)

that the component may communicate with, including virtual

segments (i.e. internal net numbers)

NType: Local Network Type.  The type of network that the component

communicates with.

┌──────┬───────────────────────────────────────────────────────┐

│NType │  Description                                          │

├──────┼───────────────────────────────────────────────────────┤

│  0   │  LAN Board                                            │

│  1   │  Non-dedicated file server (virtual internal network) │

│  2   │  Redirected Remote Line                               │

└──────┴───────────────────────────────────────────────────────┘

Net Address: The 4-byte IPX Network Address of the network that the component

communicates with.

Node Address: The 6-byte IPX Node Address that corresponds to the Net Address

given.