Novell IPX Error Protocol Packet format

─────────────────────────────────────────

┌────────────┬─────────────┬─────────────────────────┬─────────────┐

│ MAC Header │ IPX Header  │ IPX Error Protocol/Data │ MAC Trailer │

└────────────┴─────────────┴─────────────────────────┴─────────────┘

│             │                         │

┌────────────┘             └─────────────────┐       └─────────────┐

│                                            │                     │

│                ┌──── Dest ───┬──── Src ────┐                     │

┌────┬───┬──┬────┬───┬────┬────┬───┬────┬────┬─────────────────────┐

│Cksm│Len│TC│TYPE│Net│Node│Sckt│Net│Node│Sckt│        Data         │

└────┴───┴──┴────┴───┴────┴────┴───┴────┴────┴─────────────────────┘

0003                            │                     │

┌────────────────────────────────────────────┘                     │

┌──┬──┬──┬──┬──────────────────────────────────────────────────────┐

│??│??│??│??│     Error Packet's Original IPX/SPX/NCP Header       │

└──┴──┴──┴──┴──────────────────────────────────────────────────────┘

└───────────┘

Error Protocol Header

(description unavailable)

Error Protocol Packets are sent to an originating station when an error is

detected in a packet by an IPX router during packet delivery.  See the example

below:

Packet #: 25543

Length:   68 bytes

Elapsed time:      18.621 ms       Interpacket time:       0.618 ms

Receive Channels: goodPkts NetWare

802.3: ===================== IEEE 802.3 Datalink Layer =====================

Station: 08-00-09-23-21-27 ----> 08-00-0B-3D-19-79

Length:   50

ipx: =========== NetWare Internetwork Packet Exchange Protocol ===========

Checksum: 0xFFFF           Length: 50

Hop Count:  0              Packet Type: 17 (NCP)

Network: 00 00 00 10        --->  00 00 00 11

Node:    08-00-09-23-21-27  --->  00-00-00-00-00-01

Socket:  0x4003             --->  NCP

ncp: ======================= NetWare Core Protocol =======================

NCP Request: Read From a File

Request Type: 0x2222 (Request)

Sequence Number: 107

Connection Number Low: 8

Task Number: 2

Connection Number High: 0

Function Code: 72

File Handle: 0x07 0xFF 0xEB 0x10 0x00 0x00

Starting Byte Offset: 9445376

Bytes to Read: 512

Above packet in Hex:

─────────────────────

CHECKSUM: FF FF

LENGTH: 00 32

HOP COUNT: 01

PACKET TYPE: 11                     (NCP)

DEST NET: 00 00 00 11

DEST NODE: 00 00 00 00 00 01

DEST SCKT: 04 51                  (NCP)

SRC NET: 00 00 00 10

SRC NODE: 08 00 09 23 21 27

SRC SCKT: 40 03

NCP FUNCTION: 22 22                  (Read Request)

SEQ NUM: 6B

CONN# LOW: 08

TASK NUM: 02

CONN# HIGH: 00

FUNCTION CODE: 48

FILE HANDLE: 00 07 FF EB 10 00 00

START BYTE: 00 90 20 00

BYTES TO READ: 02 00

Packet #: 25544

Length:  102 bytes

Elapsed time:      19.661 ms       Interpacket time:       1.040 ms

Receive Channels: goodPkts NetWare

802.3: ===================== IEEE 802.3 Datalink Layer =====================

Station: 08-00-0B-3D-19-79 ----> 08-00-09-23-21-27

Length:   84

ipx: =========== NetWare Internetwork Packet Exchange Protocol ===========

Checksum: 0xFFFF           Length: 84

Hop Count:  0              Packet Type: 3 (Error Protocol)

Network: 00 00 00 10        --->  00 00 00 10

Node:    08-00-0B-3D-19-79  --->  08-00-09-23-21-27

Socket:  0x0003             --->  0x4003

0: 02 02 00 00 FF FF 00 32 01 11 00 00 00 11 00 00  |.......2........|

10: 00 00 00 01 04 51 00 00 00 10 08 00 09 23 21 27  |.....Q.......#!'|

20: 40 03 22 22 6B 08 02 00 48 00 07 FF EB 10 00 00  |@.""k...H.......|

30: 00 90 20 00 02 00                                |.. ...          |

(Compare data field to Packet# 25543 Hex Dump...)

2