Common Ethernet Type Field Values

───────────────────────────────────

Hex        Description

─────      ─────────────

0000-05DC *      IEEE 802.3 Length Field (0-1500 bytes)

┌                                            ┐

│ 0101-01FF        Experimental              │ Conflict with IEEE 802.3

│      0200        XEROX PUP (see 0A00)      │ Length Fields...

│      0201        PUP Addr Trans (see 0A01) │

└                                            ┘

0600        XEROX NS IDP

0800 *      DOD IP (Internet Protocol)

0801        X.75 Internet

0802        NBS Internet

0803        ECMA Internet

0804        Chaosnet

0805        X.25 Level 3

0806 *      ARP (Address Resolution Protocol)

0807        XNS Compatibility

081C        Symbolics Private

0888-088A        Xyplex

0900        Ungerman-Bass

0A00        XEROX IEEE 802.3 PUP

0A01        PUP Addr Trans

0BAD *      Banyan Systems

1000        Berkeley Trailer nego

1001-100F        Berkeley Trailer encap/IP

1600        Valid Systems

4242        PCS Basic Block Protocol

5208        BBN Simnet

6000        DEC Unassigned

6001        DEC MOP Dump/Load Assistance

6002        DEC MOP Remote Console

6003        DECnet Phase IV

6004        DEC LAT

6005        DECnet Diagnostics

6006        DECnet Customer Use

6007        DECnet LAVC/SCA

6008-6009        DEC Unassigned

6010-6014        3Com Corp

7000        Ungerman-Bass download

7001        Ungerman-Bass NIU

7002        Ungerman-Bass diag/loop

7020-7029        LRT

7030        Proteon

7034        Cabletron

8003        Cronus VLN

8004        Cronus Direct

8005        HP Probe

8006        Nestar

8008        AT&T

8010        Excelan

8013        SGI diagnostics

8014        SGI network games

8015        SGI reserved

8016        SGI bounce server

8019        Apollo Computers

802E        Tymshare

802F        Tigan Inc.

8035 *      RARP (Reverse ARP)

8036        Aeonic Systems

8038        DEC LANBridge

8039-803C        DEC Unassigned

803D        DEC Ethernet CSMA/CD Encryption Protocol

803E        DEC Unassigned

803F        DEC LAN Traffic Monitor

8040-8042        DEC Unassigned

8044        Planning Research Corp

8046        AT&T

8047        AT&T

8049        ExperData

805B        Stanford V Kernel exp.

805C        Stanford V Kernel prod.

805D        Evans & Sutherland

8060        Little Machines

8062        Counterpoint Computers

8065-8066        Univ. of Mass. @ Amherst

8067        Veeco Integrated Auto.

8068        General Dynamics

8069        AT&T

806A        Autophon

806C        ComDesign

806D        Computgraphic Corp.

806E-8077        Landmark Graphics Corp.

807A        Matra

807B        Dansk Data Elektronik

807C        Merit Internodal

807D-807F        Vitalink Communications

8080 *      Vitalink TransLAN III

8081-8083        Counterpoint Computers

8088-808A        Xyplex

809B *      Kinetics Ethertalk, Appletalk over Ethernet

809C-809E        Datability

809F        Spider Systems Ltd.

80A3        Nixdorf Commputers

80A4-80B3        Siemens Gammasonics Inc.

80C0-80C3        DCA Data Exchange Cluster

80C6        Pacer Software

80C7        Applitek Corporation

80C8-80CC        Intergraph Corp.

80CD-80CE        Harris Corp.

80CF-80D2        Taylor Instrument

80D3-80D4        Rosemount Corp.

80D5 *      IBM SNA over Ethernet

80DD        Varian Associates

80DE-80DF        Integrated Solutions

80E0-80E3        Allen-Bradley

80E4-80F0        Datability

80F2 *      Retix

80F3 *      Kinetics, Appletalk ARP (AARP)

80F4-80F5        Kinetics

80F7        Apollo Computer

80FF-8103 *      Wellfleet

8107-8109        Symbolics Private

8130        Waterloo Microsystems

8131        VG Laboratory Systems

8137-8138 *      Novell

8139-813D        KTI

814C *      SNMP

9000        Loopback (Configuration Test Protocol)

9001        3Com Bridge XNS Sys Mgmt

9002        3Com Bridge TCP/IP Sys

9003        3Com Bridge Loop Detect

FF00        BBN VITAL-LANBridge cache