Ethernet II Frame Format

──────────────────────────

(8)        (6)       (6)    (2) │<────(46-1500 bytes)────>│(4)

┌───────────┬─────────┬─────────┬────┬─────────────────────────┬───┐

Eth II  │ Preamble  │Dest Addr│ Src Addr│Type│          Data           │FCS│

└───────────┴─────────┴─────────┴────┴─────────────────────────┴───┘

└────────────────── FCS Protected ─────────────────┘

Preamble: The preamble is a 64 bit (8 byte) alternating 1010 ... pattern that

ends in ... 1011 (AA AA AA AA AA AA AA AB hex).  The preamble

provides synchronization.  The preamble does not appear on the

LANalyzer trace.  Note: The packet length does not include the

preamble.

Dest Addr: The destination address is a 48 bit (6 byte) field that denotes

either a unicast (physical) address or a multicast (group or

broadcast) address.  See the MULTICST.TXT file for a list of

common multicast addresses.  The least significant bit (LSB) of the

first byte of the destination address determines whether or not

the address is a uni- or multi- cast address.  If the LSB=0 then

it is a unicast address.  If the LSB=1, then it is a multicast

address.

byte 1  │ byte 2  │ byte 3  │ byte 4  │ byte 5  │ byte 6

─────────┼─────────┼─────────┼─────────┼─────────┼─────────

multi-  1XXX XXXX│XXXX XXXX│XXXX XXXX│XXXX XXXX│XXXX XXXX│XXXX XXXX

uni-  0XXX XXXX│XXXX XXXX│XXXX XXXX│XXXX XXXX│XXXX XXXX│XXXX XXXX

^       ^

LSB     MSB

Src Addr: The source address is a 48 bit (6 byte) field that identifies the

originating station.

Type: The type field, also know as the Ethernet Type or EtherType field,

is a 2 byte field that identifies the upper layer protocol

encapsulated in the packet's data field.  See the ETH_TYPE.TXT

file for a list of common Ethertypes.

Data: The data field ranges in size from 46 bytes to 1500 bytes.  It

contains the upper layer protocol header and it's data.  For

example, it might contain a Spanning Tree BPDU, or a Novell IPX

packet, or an IP datagram.  The upper layer protocol is responsible

for ensuring that the minimum size requirement is met.