IBM NetBIOS Frame Format

──────────────────────────

(1)(1)(1)    (6)       (6)   (0-30)│<────(variable)────>│ (4)(1)(1)

┌──┬──┬──┬─────────┬─────────┬─────┬────────────────────┬───┬──┬──┐

802.5   │SD│AC│FC│Dest Addr│ Src Addr│ RIF │       Data         │FCS│ED│FS│

└──┴──┴──┴─────────┴─────────┴─────┴────────────────────┴───┴──┴──┘

┌──────────────────────────────────┘                    └─────────┐

│(1)  (1) (1or2)<───────────────────(variable)───────────────────>│

┌────┬────┬────┬──────────────────────────────────────────────────┐

802.2   │DSAP│SSAP│Ctrl│                    Information                   │

└────┴────┴────┴──────────────────────────────────────────────────┘

│ F0   F0   03 │                                                  │

└───(LLC Hdr)──┘                                                  │

┌──────────────┘                                                  │

│(2)  (2)  (1)  (1)  (2)  (4)    (16)      (16)   │<──(variable)─>│

┌────┬────┬────┬────┬────┬────┬─────────┬─────────┬───────────────┐

│Len │EFFF│CMD │OPT1│OPT2│X/R │Dest Name│Src Name │ Optional Data │

└────┴────┴────┴────┴────┴────┴─────────┴─────────┴───────────────┘

└─────────────────(NetBIOS Header)────────────────┘               |

|                                                                 |

|                    -- or --                                     |

│                                                                 │

│(2)  (1)  (1)  (2)  (4)   (1)   (1) │<────────(variable)────────>│

┌────┬────┬────┬────┬────┬─────┬─────┬────────────────────────────┐

│Len │CMD │OPT1│OPT2│X/R │Dest#│ Src#│       Optional Data        │

└────┴────┴────┴────┴────┴─────┴─────┴────────────────────────────┘

└─────────(NetBIOS Header)───────────┘

Len:    Two byte Length field.  This is the length of the NetBIOS header

including the length field itself.

EFFF:   Two byte delimiter.  Indicates that subsequent data is destined

for the NetBIOS emulator.

CMD:    One byte NetBIOS Command field.  Commands 00 - 13 are non-session

oriented and use 802.2 Unnumbered Information (UI) frame format.

Commands 14 - 1F are session oriented and use 802.2 Information (I)

frame format.

Token Ring NetBIOS Name Management Functions

┌──────────────────────┬──────┬───────────────────────────────────────────┐

│ Command Name         │ Code │ Function                                  │

╞══════════════════════╪══════╪═══════════════════════════════════════════╡

│ Add_Group_Name_Query │ 0x00 │ Check for duplicate group name on network │

│ Add_Name_Query       │ 0x01 │ Check for duplicate name on network       │

│ Add_Name_Response    │ 0x0D │ Negative response: add name is duplicate  │

│ Name_In_Conflict     │ 0x02 │ Duplicate names detected                  │

└──────────────────────┴──────┴───────────────────────────────────────────┘

Token Ring NetBIOS Session Functions

┌──────────────────────┬──────┬───────────────────────────────────────────┐

│ Command Name         │ Code │ Function                                  │

╞══════════════════════╪══════╪═══════════════════════════════════════════╡

│ Name_Query           │ 0x0A │ Request to locate a name on the network   │

│ Name_Recognized      │ 0x0E │ Name Recognized: Name_Query response      │

│ Session_Alive        │ 0x1F │ Verify that session is still active       │

│ Session_Confirm      │ 0x17 │ Session_Initialize acknowledgement        │

│ Session_End          │ 0x18 │ Terminate session                         │

│ Session_Initialize   │ 0x19 │ A session has been set up                 │

└──────────────────────┴──────┴───────────────────────────────────────────┘

Token Ring NetBIOS Data Transfer Frames

┌──────────────────────┬──────┬───────────────────────────────────────────┐

│ Command Name         │ Code │ Function                                  │

╞══════════════════════╪══════╪═══════════════════════════════════════════╡

│ Data_Ack             │ 0x14 │ Data_Only_Last acknowledgement            │

│ Data_First_Middle    │ 0x15 │ Session data message-1st or middle frame  │

│ Datagram             │ 0x08 │ Application-generated datagram            │

│ Datagram_Broadcast   │ 0x09 │ Application-generated broadcast datagram  │

│ Data_Only_Last       │ 0x16 │ Session data message-only or last frame   │

│ No_Receive           │ 0x1A │ No receive command to hold received data  │

│ Receive_Continue     │ 0x1C │ Indicates receive outstanding             │

│ Receive_Outstanding  │ 0x1B │ Re-transmit last data - receive cmd up    │

└──────────────────────┴──────┴───────────────────────────────────────────┘

Token Ring NetBIOS Additional Frames

┌──────────────────────┬──────┬───────────────────────────────────────────┐

│ Command Name         │ Code │ Function                                  │

╞══════════════════════╪══════╪═══════════════════════════════════════════╡

│ Status_Query         │ 0x03 │ Request remote node status                │

│ Status_Response      │ 0x0F │ Remote node status information            │

│ Terminate_Trace      │ 0x07 │ Terminate traces at remote nodes          │

│ Terminate_Trace      │ 0x13 │ Terminate traces at local and remote nodes│

└──────────────────────┴──────┴───────────────────────────────────────────┘

OPT1:   One byte optional data field for certain commands.

OPT2:   An additional two byte optional data field used by certain commands.

X/R:    Four byte correlator used to associate received responses with

transmitted requests.

Dest Name:

Sixteen byte destination station name for non-session oriented

frames.  The name is in ascii.

Src Name:

Sixteen byte source station name for non-session oriented frames.

The name is in ascii.

Dest#:  One byte destination station number for session oriented frames.

Src#:   One byte source station number for session oriented frames.

Optional Data:

Depending upon the command, this field may be blank, may contain

data or may conatin the Server Message Block (SMB) protocol

header.

Example Token Ring NetBIOS frame

──────────────────────────────────

Packet #: 6

Length:   81 bytes

Elapsed time:       0.000 ms       Interpacket time:       0.000 ms

Receive Channels: GoodPkts Promiscu

802.5: ===================== IEEE 802.5 Datalink Layer =====================

AC: Frame Priority=0  Monitor Count=1  Priority Reservation=0

FC: Non-MAC Frame     Attention Code=0

Station: 10-00-5A-8D-E3-13 -----> C0-00-00-00-00-80

Source Routing Information:

Length:         16 bytes

Broadcast:      Single-route broadcast, all routes broadcast return

Direction:      From originating station

Largest Frames: 1500 bytes (LF=1)

Ring Number     9   2006  3006  2001  3011  2011  12

|   ^ |   ^ |   ^ |   ^ |   ^ |   ^

Bridge Number   |-1-| |-1-| |-1-| |-1-| |-1-| |-1-|

802.2: ================== IEEE 802.2 Logical Link Control ==================

SSAP: NetBEUI     DSAP: NetBEUI

Unnumbered Command: Unnumbered Information (UI)

nbeui: ============================== NetBEUI ==============================

Command: 0x03 (Status Query)

Type of Control: CONNECTIONLESS Control

Header length: 44

Response Correlator Value: 0x4738

Sending Node Permanent Name: .............Z..

Name of Receiver: DOS1FC         .

Additional Information:

Initial Status Request NETBIOS 2.1

Length of user's status buffer: 348