Internet Control Message Protocol (ICMP) Frame Formats

────────────────────────────────────────────────────────

ICMP General Format

───────────────────

┌────────────┬───────────┬────────────┬──────────────────┬─────────────┐

│ MAC Header │ IP Header │ UDP Header │ ICMP Header/Data │ MAC Trailer │

└────────────┴───────────┴────────────┴──────────────────┴─────────────┘

│                  │

┌──────────────────────────────────┘                  └─────────┐

│0               8              16              24            31│

┌───────────────┼───────────────┼───────────────┴───────────────┐

│     Type      │     Code      │           Checksum            │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                      ICMP Message Data...                     │

└───────────────────────────────────────────────────────────────┘

TYPE field:

┌───────────┬─────────────────────────────────────┐

│ TYPE (dec)│         ICMP Message Type           │

├───────────┼─────────────────────────────────────┤

│     0     │   Echo reply                        │

│     3     │   Destination unreachable           │

│     4     │   Source quench                     │

│     5     │   Redirect (change route)           │

│     8     │   Echo request                      │

│    11     │   Time exceeded for datagram        │

│    12     │   Parameter problem on datagram     │

│    13     │   Timestamp request                 │

│    14     │   Timestamp reply                   │

│    15     │   Information request (obsolete)    │

│    16     │   Information reply (obsolete)      │

│    17     │   Address mask request              │

│    18     │   Address mask reply                │

└───────────┴─────────────────────────────────────┘

CODE Field: Varies depending upon value of TYPE field.

ICMP Datagram Formats

───────────────────────

ECHO REPLY (Type=0)

┌───────────────────────────────────────────────────────────────────┐

│ 0                8              16              24            31  │

│ ┌───────────────┼───────────────┼───────────────┴───────────────┐ │

│ │     Type (0)  │     Code (0)  │           Checksum            │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │          Identifier           │       Sequence Number         │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                         Optional Data                         │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                              ...                              │ │

│ └─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘ │

│  The ICMP Echo Reply datagram is sent in response to an ICMP      │

│  Echo Request.  The process is more commonly called a "ping".     │

│  See the Echo Request (Type 8) format below.                      │

└───────────────────────────────────────────────────────────────────┘

DESTINATION UNREACHABLE (Type=3)

┌───────────────────────────────────────────────────────────────────┐

│ 0                8              16              24            31  │

│ ┌───────────────┼───────────────┼───────────────┴───────────────┐ │

│ │     Type (3)  │  Code (0-12)  │           Checksum            │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                    Unused (must be zero)                      │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │        Internet Header + first 64 bits of datagram            │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                              ...                              │ │

│ └─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘ │

│                                                                   │

│    ┌────────────────┬────────────────────────────────────────┐    │

│    │   CODE field   │            Description                 │    │

│    ├────────────────┼────────────────────────────────────────┤    │

│    │       0        │   Network Unreachable                  │    │

│    │       1        │   Host Unreachable                     │    │

│    │       2        │   Protocol Unreachable                 │    │

│    │       3        │   Port Unreachable                     │    │

│    │       4        │   Fragmentation needed and DF set      │    │

│    │       5        │   Source route failed                  │    │

│    │       6        │   Destination network unknown          │    │

│    │       7        │   Destination host unknown             │    │

│    │       8        │   Source host isolated                 │    │

│    │       9        │   Communication with destination       │    │

│    │                │   net administratively prohibited      │    │

│    │      10        │   Communication with destination       │    │

│    │      11        │   host administratively prohibited     │    │

│    │      12        │   Host unreachable for type of service │    │

│    └────────────────┴────────────────────────────────────────┘    │

└───────────────────────────────────────────────────────────────────┘

SOURCE QUENCH (Type=4)

┌───────────────────────────────────────────────────────────────────┐

│ 0                8              16              24            31  │

│ ┌───────────────┼───────────────┼───────────────┴───────────────┐ │

│ │     Type (4)  │  Code (0)     │           Checksum            │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                    Unused (must be zero)                      │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │     Internet Header + first 64 bits of original datagram      │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                              ...                              │ │

│ └─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘ │

│  The ICMP Source Quench message is used by IP routers to reduce   │

│  network congestion.  If a device, or devices, are generating     │

│  traffic faster than the router can process the traffic, packets  │

│  may be discarded.  The router may send a Source Quench message   │

│  to the sending device requesting that it reduce it's rate of     │

│  datagram transmission.  Congestion occurs for many reasons. Most │

│  commonly, it occurs when a large amount of traffic from a high   │

│  speed LAN needs to cross a low speed WAN link.  The router that  │

│  makes the LAN-WAN connection experiences congestion when it's    │

│  capacity to buffer incoming traffic is exceded.                  │

└───────────────────────────────────────────────────────────────────┘

REDIRECT (Type=5)

┌───────────────────────────────────────────────────────────────────┐

│ 0                8              16              24            31  │

│ ┌───────────────┼───────────────┼───────────────┴───────────────┐ │

│ │     Type (5)  │  Code (0-3)   │           Checksum            │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                 Gateway Internet Address                      │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │     Internet Header + first 64 bits of original datagram      │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                              ...                              │ │

│ └─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘ │

│                                                                   │

│ ┌──────┬────────────────────────────────────────────────────────┐ │

│ │ Code │               Meaning                                  │ │

│ ├──────┼────────────────────────────────────────────────────────┤ │

│ │  0   │ Redirect Datagrams for the Net (obsolete)              │ │

│ │  1   │ Redirect Datagrams for the Host                        │ │

│ │  2   │ Redirect Datagrams for the Type of Service and Net     │ │

│ │  3   │ Redirect Datagrams for the Type of Service and Host    │ │

│ └──────┴────────────────────────────────────────────────────────┘ │

│                                                                   │

│  If an IP Host on a directly connected network sends a datagram   │

│  to an IP router for delivery to a device on another network, and │

│  the router knows that a better route to the destination network  │

│  exists, then the IP router will send an ICMP Redirect message to │

│  the Host to notify it of the preferred route.  The Gateway       │

│  Internet Address in the Redirect message is the IP address of    │

│  the router that the Host should use to reach the destination     │

│  network.  Note that router forwards the original packet before   │

│  the Redirect message is sent.                                    │

└───────────────────────────────────────────────────────────────────┘

ECHO REQUEST (Type=8)

┌───────────────────────────────────────────────────────────────────┐

│ 0                8              16              24            31  │

│ ┌───────────────┼───────────────┼───────────────┴───────────────┐ │

│ │     Type (8)  │     Code (0)  │           Checksum            │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │          Identifier           │       Sequence Number         │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                         Optional Data                         │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                              ...                              │ │

│ └─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘ │

│  The ICMP Echo Request datagram is used to test connectivity.     │

│  An Echo Request is sent to a remote device and the remote        │

│  device, if active, returns an Echo Reply datagram.               │

└───────────────────────────────────────────────────────────────────┘

TIME EXCEEDED (Type=11)

┌───────────────────────────────────────────────────────────────────┐

│ 0                8              16              24            31  │

│ ┌───────────────┼───────────────┼───────────────┴───────────────┐ │

│ │   Type (11)   │  Code (0-1)   │           Checksum            │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                    Unused (must be zero)                      │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │        Internet Header + first 64 bits of datagram            │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                              ...                              │ │

│ └─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘ │

│                                                                   │

│ ┌──────┬────────────────────────────────────────────────────────┐ │

│ │ Code │       Meaning                                          │ │

│ ├──────┼────────────────────────────────────────────────────────┤ │

│ │  0   │ Time-To-Live exceeded                                  │ │

│ │  1   │ Fragment Reassembly Time Exceeded                      │ │

│ └──────┴────────────────────────────────────────────────────────┘ │

│                                                                   │

│  Time To Live Exceeded:                                           │

│                                                                   │

│  When a route is lost on an internet, the fact that the route has │

│  been lost is not immediately known to all routers on the         │

│  internet becaus routing information updates take time to         │

│  propagate across the network.  During that time, it is possible  │

│  for routing loops to exist.  A routing loop could cause a packet │

│  to circle the internet endlessly, being forwarded from one router│

│  to the next, never able to reach it's destination.  For this     │

│  reason, every IP datagram that is transmitted has a Time-to-Live │

│  field in the IP header that specifies how long the datagram      │

│  should be allowed to exist on the internet.  As the datagram is  │

│  processed by a router, the TTL field is decremented.  If the TTL │

│  field reaches zero, the packet is discarded by the router.  The  │

│  router then sends a Time To Live Exceeded message to the         │

│  originating Host.                                                │

│                                                                   │

│  Fragment Reassembly Time Exceeded:                               │

│                                                                   │

│  When a datagram is fragmented during transit through the internet│

│  the destination host must collect the fragments and reassemble   │

│  the datagram.  When the first fragment of the datagram arrives,  │

│  the receiving host starts a timer and considers it an error if   │

│  the timer expires before the entire packet is reassembled.  When │

│  this occurs, the destination host sends an ICMP Fragment         │

│  Reassembly Time Exceeded message to the originating host and     │

│  discards the fragment that it received before the timer expired. │

└───────────────────────────────────────────────────────────────────┘

PARAMETER PROBLEM (Type=12)

┌───────────────────────────────────────────────────────────────────┐

│ 0                8              16              24            31  │

│ ┌───────────────┼───────────────┼───────────────┴───────────────┐ │

│ │   Type (12)   │  Code (0-1)   │           Checksum            │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │    Pointer    │          Unused (must be zero)                │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │        Internet Header + first 64 bits of datagram            │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                              ...                              │ │

│ └─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘ │

│                                                                   │

│ ┌──────┬────────────────────────────────────────────────────────┐ │

│ │ Code │                    Meaning                             │ │

│ ├──────┼────────────────────────────────────────────────────────┤ │

│ │  0   │  An incorrect IP Datagram Header was received.         │ │

│ │  1   │  A Required Option was missing in the received         │ │

│ │      │  IP datagram.                                          │ │

│ └──────┴────────────────────────────────────────────────────────┘ │

│                                                                   │

│  Pointer: The pointer is used by the CODE 0 option to indicate    │

│           which byte of the IP Datagram Header was incorrect.     │

│                                                                   │

│  The destination host uses the ICMP Parameter Problem message     │

│  to notify the sending host that a datagram was received that     │

│  contained an incorrect value or option in the IP Header.         │

└───────────────────────────────────────────────────────────────────┘

TIMESTAMP REQUEST/REPLY (Type=13/14)

┌───────────────────────────────────────────────────────────────────┐

│ 0                8              16              24            31  │

│ ┌───────────────┼───────────────┼───────────────┴───────────────┐ │

│ │ Type (13/14)  │  Code (0)     │           Checksum            │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │           Identifier          │       Sequence Number         │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                      Originate Timestamp                      │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                        Receive Timestamp                      │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                       Transmit Timestamp                      │ │

│ └─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘ │

│                                                                   │

│  Type 13 = Request                                                │

│  Type 14 = Reply                                                  │

│                                                                   │

│  Originate Timestamp: Is filled in by the original sender just    │

│  before the packet is transmitted.                                │

│                                                                   │

│  Receive Timestamp: Is filled immediately upon receipt of a       │

│  request.                                                         │

│                                                                   │

│  Transmit Timestamp: Is filled immmediately before the reply is   │

│  transmitted.                                                     │

│                                                                   │

│  Hosts use the three timestamp fields to compute estimates of the │

│  delay time between them and synchronize their clocks.            │

└───────────────────────────────────────────────────────────────────┘

TIMESTAMP REQUEST/REPLY (Type=17/18)

┌───────────────────────────────────────────────────────────────────┐

│ 0                8              16              24            31  │

│ ┌───────────────┼───────────────┼───────────────┴───────────────┐ │

│ │ Type (17/18)  │  Code (0)     │           Checksum            │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │           Identifier          │       Sequence Number         │ │

│ ├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤ │

│ │                          Address Mask                         │ │

│ └─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘ │

│                                                                   │

│  Type 17=Address Mask Request                                     │

│  Type 18=Address Mask Reply                                       │

└───────────────────────────────────────────────────────────────────┘