IEEE 802.3 Frame Format

────────────────────────-

(7)    (1)    (6)       (6)    (2)│<────(46-1500 bytes)────>│(4)

┌────────┬───┬─────────┬─────────┬───┬─────────────────────────┬───┐

802.3   │Preamble│SFD│Dest Addr│ Src Addr│Len│        Data     |  Pad  │FCS│

└────────┴───┴─────────┴─────────┴───┴─────────────────────────┴───┘

└───────────────── FCS Protected ─────────────────┘

Preamble: The preamble is a 56 bit (7 byte) alternating 1010 ... pattern that

provides synchronization. In hex it appears as AA AA AA AA AA AA AA.

The LANalyzer does not display the Preamble or the SFD.

SFD: The start frame delimiter indicates the beginning of the frame.

It is an 8 bit 10101011 pattern ("AB" hex).  The 802.3 preamble,

taken together with the SFD are equivalent to the Ethernet II

preamble.

Dest Addr: The destination address is a 48 bit (6 byte) field.  The

individual/group (I/G) field specifies whether the destination

is an individual (physical) address, or a group (broadcast)

address.  If I/G=0 then it is an individual address.  If the I/G=1,

then it is a group address.  The universal/local (U/L) field

indicates whether the address is administered universally, by the

IEEE, or locally, by the end user.

Destination Address field:

┌─────┬─────┬──────────────────────────────────────────────┐

│ I/G │ U/L │                    46 bits                   │

└─────┴─────┴──────────────────────────────────────────────┘

LSB                                                  MSB

Src Addr: The source address is a 48 bit (6 byte) field that identifies the

originating station.

Length: The length field is two bytes long and indicates the number of

bytes in the data field.

Data: The data field ranges in size from 46 bytes to 1500 bytes.  It

contains the upper layer protocol header and it's data.  The data

field should contain an IEEE 802.2 LLC header followed by some

upper layer protocol.  Novell IPX is an exception.  In certain

configurations, IPX does not use an 802.2 header.  In this case,

the IPX header immediately follows the 802.3 header.

See PROTOCOL\IEEE8022.TXT and PROTOCOL\IPX.TXT for details.

Pad: The pad field ensures that the data field complies with the 46 byte

minimum length requirement.  Unlike Ethernet II, 802.3 does not

rely upon the upper layer protocol to ensure that the data field

is at least 46 bytes long.

FCS: The frame check sequence field is a 32 bit cyclic redundancy

check (CRC) calculated from the Dest Addr, Src Addr, Length, Data,

and Pad fields.

IEEE 802.3 MAC frame with IEEE 802.2 LLC header

─────────────────────────────────────────────────

┌─────────────────────────────────────────────────────────────────────────────┐

│           (7)    (1)    (6)       (6)    (2)│<────(46-1500 bytes)────>│(4)  │

│        ┌────────┬───┬─────────┬─────────┬───┬─────────────────────────┬───┐ │

│802.3   │Preamble│SFD│Dest Addr│ Src Addr│Len│        Data     |  Pad  │FCS│ │

│        └────────┴───┴─────────┴─────────┴───┴─────────────────────────┴───┘ │

│        ┌────────────────────────────────────┘                         └───┐ │

│        ┌────┬────┬────┬───────────────────────────────────────────────────┐ │

│802.2   │DSAP│SSAP│Ctrl│                   Information                     │ │

│        └────┴────┴────┴───────────────────────────────────────────────────┘ │

│         (1)  (1) (1or2)<────────────────(0 - 1497 bytes)─────────────────>│ │

│        └─── LLC Hdr ──┘                                                     │

└─────────────────────────────────────────────────────────────────────────────┘