IEEE 802.5 Token Ring Frame Format

────────────────────────────────────

(1)(1)(1)    (6)       (6)  (0-30)       (variable)        (4)(1)(1)

┌──┬──┬──┬─────────┬─────────┬───┬────────────────────────┬───┬──┬──┐

│SD│AC│FC│Dest Addr│ Src Addr│RIF│         Data           │FCS│ED│FS│

└──┴──┴──┴─────────┴─────────┴───┴────────────────────────┴───┴──┴──┘

│  └──────────────────── FCS Protected ────────────────┘

└──────────── Code Violation Protected ────────────────┘

┌─┬─┬─┬─┬─┬─┬─┬─┐

SD: │J│K│0│J│K│0│0│0│  Starting Delimiter

└─┴─┴─┴─┴─┴─┴─┴─┘

J = non-data pattern j

K = non-data pattern k

(see c:\seminar\files\info\physical\encoding.txt for details

concerning "j" and "k" bits.)

0             7

┌─┬─┬─┬─┬─┬─┬─┬─┐

AC: │P│P│P│T│M│R│R│R│  Access Control

└─┴─┴─┴─┴─┴─┴─┴─┘

PPP = priority bits (bits 0 - 2)

default priority = 011 = (3)

T = token bit

0 = Frame is a token

1 = Frame contains data

M = monitor bit

0 = Frame not seen by active monitor yet.

1 = Frame has been seen by active monitor.

RRR = priority reservation bits

Used to request a token of a certain priority.

0             7

┌─┬─┬─┬─┬─┬─┬─┬─┐

FC: │F│F│0│0│Z│Z│Z│Z│  Frame Control

└─┴─┴─┴─┴─┴─┴─┴─┘

FF = frame type (bits 0-1)

┌────┬───────────────────────┐

│ FF │ Frame Type            │

├────┼───────────────────────┤

│ 00 │ MAC control frame     │

│ 01 │ Non-MAC control frame │

│ 10 │ reserved              │

│ 11 │ reserved              │

└────┴───────────────────────┘

00 = Always set to 0 (bits 2-3)

ZZZZ = Physical Control Field (PCF) (bits 4-7)

┌──────┬─────────────────────────┐

│ PCF  │ Description             │

├──────┼─────────────────────────┤

│ 0000 │ Normal Buffer           │

│ 0001 │ Express Buffer          │

│ 0010 │ Beacon                  │

│ 0011 │ Claim Token             │

│ 0100 │ Ring Purge              │

│ 0101 │ Active Monitor Present  │

│ 0110 │ Standby Monitor Present │

└──────┴─────────────────────────┘

┌─┬─┬──────────────┬─┬───────────────────────────────┐

DA: │ │ │ 14-bit Ring# │ │      31-bit Station#          │

└─┴─┴──────────────┴─┴───────────────────────────────┘

│ │                │

│ │                └> Non-functional(1) / Functional(0) address

│ │                   (group and broadcast only)

│ │

│ └> Local(1) / Univeral(0) address

└──> Group(1) / Individual(0) address

Broadcast Address: C000FFFFFFFF or FFFFFFFFFFFF

┌─┬─┬──────────────┬─┬───────────────────────────────┐

│ │ │11111111111111│ │                               │

│1│1│     or       │1│1111111111111111111111111111111│

│ │ │00000000000000│ │                               │

└─┴─┴──────────────┴─┴───────────────────────────────┘

Functional Address:

┌─┬─┬──────────────┬─┬───────────────────────────────┐

│1│1│00000000000000│0│    Functional Address         │

└─┴─┴──────────────┴─┴───────────────────────────────┘

└> Functional

┌──────────────────────┬────────────────────────┐

│ Functional Address   │  Description           │

├──────────────────────┼────────────────────────┤

│ C0 00 00 00 00 01    │  Active Monitor        │

│ C0 00 00 00 00 02    │  Ring Parameter Server │

│ C0 00 00 00 00 08    │  Ring Error Monitor    │

│ C0 00 00 00 00 10    │  Network Manager       │

│ C0 00 00 00 00 80    │  NetBIOS               │

│ C0 00 00 00 01 00    │  Bridge                │

└──────────────────────┴────────────────────────┘

Group Address:

┌─┬─┬──────────────┬─┬───────────────────────────────┐

│1│1│00000000000000│1│      Group Address            │

└─┴─┴──────────────┴─┴───────────────────────────────┘

└> Non-Functional (group)

│<─────── 3 bytes ────────>│<────── 3 bytes ───────>│

┌─┬─┬──────────────────────┬────────────────────────┐

SA: │X│X│<──── Vendor ID ─────>│<──── Node Address ────>│

└─┴─┴──────────────────────┴────────────────────────┘

│ └> 0=IEEE Administered, 1=Locally Administered

└──> Routing Information Indicator, 1=Source Routed

┌────────────────┬────────────────┬─────────────────┐

RIF: │ Route Control  │   Segment 1    │   Segment N     │

└────────────────┴────────────────┴─────────────────┘

│                │                └───────────────┐

│                └──────────────┐                 │

│                               │                 │

┌─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬────────────┬────┐

│B│B│B│L│L│L│L│L│D│S│S│S│S│r│r│r│RRRRRRRRRRRR│BBBB│

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴────────────┴────┘

Route Control

───────────────

BBB: Broadcast type

┌─────┬─────────────────────────────────────────────┐

│Value│ Source Route Frame Type                     │

├─────┼─────────────────────────────────────────────┤

│ 000 │ Non-bdcst                                   │

│ 100 │ All routes bdcst, non-bdcst return          │

│ 110 │ Single-route bdcst, non-bdcst return        │

│ 111 │ Single-route bdcst, all-routes bdcst return │

└─────┴─────────────────────────────────────────────┘

LLLLL: Length of RIF field (including control field)

D: Direction bit

0 = Origin───> Target

1 = Target───> Origin

SSSS: Largest Frame Size

┌──────┬───────────────────────────────────┐

│Value │ Frame Size                        │

├──────┼───────────────────────────────────┤

│ 0001 │ 516 bytes                         │

│ 0011 │ 1500 bytes                        │

│ 0101 │ 2052 bytes                        │

│ 0111 │ 4472 bytes                        │

│ 1111 │ Initial value of broadcast frames │

└──────┴───────────────────────────────────┘

rrr: Reserved (set to 000)

Segment n

───────────

R: Ring Number (3 hex digits)

┌─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┐

│R│R│R│R│R│R│R│R│R│R│R│R│

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘

└───────┴───────┴───────┘

X       X       X

B: Bridge Number (1 hex digit)

┌─┬─┬─┬─┐

│B│B│B│B│

└─┴─┴─┴─┘

└───────┘

X

┌────────┬────────┬────────┬────────┐

FCS: │xxxxxxxx│xxxxxxxx│xxxxxxxx│xxxxxxxx│  Frame Check Sequence

└────────┴────────┴────────┴────────┘

┌─┬─┬─┬─┬─┬─┬─┬─┐

ED: │J│K│1│J│K│1│I│E│  Ending Delimiter

└─┴─┴─┴─┴─┴─┴─┴─┘

J = Non-data pattern J

K = Non-data pattern K

I = Intermediate Frame

0 = Last or only frame

1 = More frames coming

E = Error Detected Indicator

0 = No Error

1 = Error

┌─┬─┬─┬─┬─┬─┬─┬─┐

FS: │A│F│0│0│A│F│0│0│  Frame Status

└─┴─┴─┴─┴─┴─┴─┴─┘

A = Address Recognized Indicator (ARI)

0 = Not Recognized

1 = Recognized

F = Frame Copied Indicator (FCI)

0 = Not Copied

1 = Copied

Note: The ARI and FCI bits are duplicated in the Frame Status

field because the the field is neither FCS nor Code

Violation protected.

Special Frame Types

─────────────────────

┌──┬──┬──┐

│SD│AC│ED│   Token

└──┴──┴──┘

┌──┬──┐

│SD│ED│      Abort Sequence

└──┴──┘

IEEE 802.5 MAC frame with IEEE 802.2 LLC header

─────────────────────────────────────────────────

┌─────────────────────────────────────────────────────────────────────────────┐

│        (1)(1)(1)    (6)       (6)   (0-30)│<────(variable)─────>│ (4)(1)(1) │

│        ┌──┬──┬──┬─────────┬─────────┬─────┬─────────────────────┬───┬──┬──┐ │

│802.5   │SD│AC│FC│Dest Addr│ Src Addr│ RIF │       Data          │FCS│ED│FS│ │

│        └──┴──┴──┴─────────┴─────────┴─────┴─────────────────────┴───┴──┴──┘ │

│              ┌────────────────────────────┘                     │           │

│              ┌────┬────┬────┬───────────────────────────────────┐           │

│802.2         │DSAP│SSAP│Ctrl│            Information            │           │

│              └────┴────┴────┴───────────────────────────────────┘           │

│               (1)  (1) (1or2)<────────(variable length)────────>│           │

│              └─── LLC Hdr ──┘                                               │

└─────────────────────────────────────────────────────────────────────────────┘

NOTE: See c:\seminar\files\info\datalink\macframe.txt for a description of

MAC frame formats.