Internet Protocol (IP) Packet Structure

─────────────────────────────────────────

┌─────────────┬───────────┬─────────────────────┬───────────────┐

│ MAC Header  │ IP Header │ Layer 4 Header/Data │  MAC Trailer  │

└─────────────┴───────────┴─────────────────────┴───────────────┘

┌─────────────┘           └─────────────────────────────────────┐

0                8              16              24            31  (bits)

┌───────┬───────┼───────────────┼───────────────┴───────────────┐

│Version│  IHL  │Type of Service│        Total Length           │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┬─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│        Identification         │flags│   Fragment Offset       │

├─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│Time to Live   │  Protocol     │       Header Checksum         │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                      Source IP Address                        │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                    Destination IP Address                     │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┤

│                 Options (variable)            |   Padding     │

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘

Version: Indicates the format of the IP Header.  Version 4 is the

current version.

IHL: Internet Header Length.  Length is specified in units of

32 bits.  (range: 5 - 15)

Type of Service: Parameters describing quality of service; precedence, delay

throughput, reliability.

Total Length: Total length of datagram in bytes, including the IP Header

and data.

Identification: Fragment identification.  Similar to a sequence number.

Flags: Control Flags... "Don't/May Fragment" and "More/Last Fragment"

See fragmentation example below.

Fragment Offset: Indicates the position of the fragment's data, in bytes.

Time to Live: Maximum time, in seconds, that the datagram may remain in

the internet.  Intermediate routers decrement this field as

the datagram makes it's way through the network.

Protocol: Indicates which upper layer protocol is encapsulated in the

data portion of the datagram.  See FILES\INFO\IP_PROT.TXT

for list of assigned protocols.

Header Checksum: Checksum of the IP Header.

Source Address: Internet address of the originating host.

Dest Address: Internet address of the datagram's target host.

Options: Not required...used primarily for network debugging.

See full details below...

Padding: Ensures that the IP header ends on a 32-bit boundry.

IP OPTION FIELD GENERAL FORMAT

────────────────────────────────

0                                  31

┌────────┬────────┬────────┬────────┐

│  CODE  │ Length │Pointer │  ***   │

├────────┴────────┴────────┴────────┤

│          Option data...           │

├───────────────────────────────────┤

│               ...                 │

└───────────────────────────────────┘

CODE:  One byte field defining the option class and option number

┌─────────────────────────────────────────────────────────────────────────┐

│     0       1       2       3       4       5       6       7           │

│ ┌───────┬───────┬───────┬───────┬───────┬───────┬───────┬───────┐       │

│ │ COPY  │ OPTION CLASS  │             OPTION NUMBER             │       │

│ └───────┴───────┴───────┴───────┴───────┴───────┴───────┴───────┘       │

│                                                                         │

│  COPY:  0 = Copy option field to first fragment header only.            │

│         1 = Copy option field into all fragment headers if              │

│             fragmentation is required.                                  │

│                                                                         │

│  OPTION CLASS:  0 = Datagram or Network Control                         │

│                 1 = reserved for future use                             │

│                 2 = Debugging and Measurement                           │

│                 3 = reserved for future use                             │

│                                                                         │

│  OPTION NUMBER:                                                         │

│ ┌───────┬────────┬─────┬──────────────────────────────────────────────┐ │

│ │ Class │ Option │ Len │                Description                   │ │

│ ╞═══════╪════════╪═════╪══════════════════════════════════════════════╡ │

│ │   0   │   0    │  -  │ End of option list.  Used if options do not  │ │

│ │       │        │     │ end at end of header field.                  │ │

│ ├───────┼────────┼─────┼──────────────────────────────────────────────┤ │

│ │   0   │   1    │  -  │ No Operation.  Used to align bytes in a list │ │

│ │       │        │     │ of options.                                  │ │

│ ├───────┼────────┼─────┼──────────────────────────────────────────────┤ │

│ │   0   │   2    │ 11  │ Security and handling restrictions.          │ │

│ │       │        │     │ (for military applications)                  │ │

│ ├───────┼────────┼─────┼──────────────────────────────────────────────┤ │

│ │   0   │   3    │ var │ Loose Source Routing.  Used to route a       │ │

│ │       │        │     │ datagram along a specified path.             │ │

│ ├───────┼────────┼─────┼──────────────────────────────────────────────┤ │

│ │   0   │   7    │ var │ Record Route.  Used to trace the route that  │ │

│ │       │        │     │ a datagram takes.                            │ │

│ ├───────┼────────┼─────┼──────────────────────────────────────────────┤ │

│ │   0   │   8    │  4  │ Stream identifier.  Used to carry a SATNET   │ │

│ │       │        │     │ stream identifier (obsolete).                │ │

│ ├───────┼────────┼─────┼──────────────────────────────────────────────┤ │

│ │   0   │   9    │ var │ Strict Source Routing.  Used to route aa     │ │

│ │       │        │     │ datagram along a specified path.             │ │

│ ├───────┼────────┼─────┼──────────────────────────────────────────────┤ │

│ │   2   │   4    │ var │ Internet Timestamp.  Used to record time-    │ │

│ │       │        │     │ stamps along the route.                      │ │

│ └───────┴────────┴─────┴──────────────────────────────────────────────┘ │

└─────────────────────────────────────────────────────────────────────────┘

LENGTH:  Total length of the IP datagram's OPTION field.

POINTER:  The POINTER field acts as an index pointer for the Option Data

field.  For the Source Route and Record Route Options, the

option data field contains a list of IP Addresses.  The POINTER

is an offset pointing to a particular address in the option data

field.

*** :  Used by the Timestamp Option.  See below for details.

OPTION DATA:  Varies depending upon option.

Loose Source Route Option

───────────────────────────

0                                  31

┌────────┬────────┬────────┬────────┐

│CODE 131│ Length │Pointer │ unused │

├────────┴────────┴────────┴────────┤

│      IP Address of First Hop      │

├───────────────────────────────────┤

│     IP Address of Second Hop      │

├───────────────────────────────────┤

│               ...                 │

└───────────────────────────────────┘

Loose Source Routing allows the originating station to specify the path

that the datagram must take through the network.  With loose source

routing, not all of the intermediate hops have to be specified.  If five

routers must be crossed to reach the destination network, the source

station might specify three of the five.  If alternate routes exist for

the intermediate hops not specified, then the network's routing algorithms

will determine the best route to take for those hops.

Strict Source Route Option

────────────────────────────

0                                  31

┌────────┬────────┬────────┬────────┐

│CODE 137│ Length │Pointer │ unused │

├────────┴────────┴────────┴────────┤

│      IP Address of First Hop      │

├───────────────────────────────────┤

│     IP Address of Second Hop      │

├───────────────────────────────────┤

│               ...                 │

└───────────────────────────────────┘

The Strict Source Routing Option is similar to the loose source routing

option described above, except that ALL intermediate hops must be

specified.

Record Route Option

─────────────────────

0                                  31

┌────────┬────────┬────────┬────────┐

│ CODE 7 │ Length │Pointer │ unused │

├────────┴────────┴────────┴────────┤

│          First IP Address         │

├───────────────────────────────────┤

│          Second IP Address        │

├───────────────────────────────────┤

│               ...                 │

└───────────────────────────────────┘

The Record Route Option forces intermediate routers to record the route

taken by the datagram.  Note that the source device does not automatically

receive information about how the datagram was routed.  The destination

application is responsible for processing the resultant list.

The routing options described above are useful for troubleshooting

IP networks.  Some versions of the "ping" application (FTP's PC/TCP

software for example) allow the routing options to be specified.

A remote Host can be pinged with the record route option to determine

the path taken.  The source route options can be used to specify a

particular route for the datagram and the round-trip delivery time for

various routes can be recorded.  Network congestion and other problems

can be detected in this manner.  In addition, applications that support

the source routing options can be configured to avoid certain network

paths.  This is useful if a new path is being brought on-line and is

still under testing or if certain paths are to be restricted for traffic

or security reasons.

Timestamp Option

──────────────────

0                                  31

┌────────┬────────┬────────┬────┬────┐

│CODE 68 │ Length │Pointer │OvFl Flag│

├────────┴────────┴────────┴────┴────┤

│          First IP Address          │

├────────────────────────────────────┤

│           First Timestamp          │

├────────────────────────────────────┤

│               ...                  │

└────────────────────────────────────┘

OvFl:  Four bit count of routers that could not supply a timestamp

because the option field was too small.

Flag:  Specifies how routers should supply the timestamp.

0 = Record Timestamps only; Omit IP Addresses

1 = Precede each Timestamp by an IP Address (this is the format

shown above.

3 = IP Addresses are specified by the sender.  A router only

records a Timestamp if the next IP Address in the list matches

the router's IP Address.

Timestamps are in universal time if available.  If unavailable, the router

will use it's own intternal representation of local time.

Fragmentation Example

───────────────────────

If the length of the IP datagram excedes the Maximum Transfer Unit

(MTU) size of an intermediate network that must be traversed to

reach the destination, the packet will be fragmented.  In the example

below, the original datagram contains a 1400 byte data field.  The

MTU of an intermediate network is 500 bytes:

Original datagram:

┌──────────────────┬────────────────────────────────────────────────────┐

│ Datagram Header  │                 data (1400 bytes)                  │

└──────────────────┴────────────────────────────────────────────────────┘

┌──────────────────┬──────────────────┐

│ Fragment1 Header │      data1       │    500 bytes (Offset=0)

└──────────────────┴──────────────────┘

┌──────────────────┬──────────────────┐

│ Fragment2 Header │      data2       │    500 bytes (Offset=500)

└──────────────────┴──────────────────┘

┌──────────────────┬──────────────┐

│ Fragment3 Header │   data3      │        400 bytes (Offset=1000)

└──────────────────┴──────────────┘

The Fragment Header duplicates most of the original Datagram Header,

except for the Fragment flag and Fragment offset field.  The datagram

is reassembled by the destination station.  When an station receives

an initial fragment, it starts a fragmentation timer.  If the timer

expires before all of the fragments are received, the fragments are

all discarded.  The probability of datagram loss increases when a

packet must be fragmented because the entire packet will be discarded

if any of the fragments is lost.

Internet Protocol Ethernet II Frame Format

────────────────────────────────────────────

(8)        (6)       (6)    (2) │<────(46-1500 bytes)────>│(4)

┌───────────┬─────────┬─────────┬────┬─────────────────────────┬───┐

Eth II  │ Preamble  │Dest Addr│ Src Addr│Type│          Data           │FCS│

└───────────┴─────────┴─────────┴────┴─────────────────────────┴───┘

0800 │                         │

┌─────────────────────────────────┘                         └───┐

│0               8              16              24            31│

┌───────┬───────┼───────────────┼───────────────┴───────────────┐

│Version│  IHL  │Type of Service│        Total Length           │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┬─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│        Identification         │flags│   Fragment Offset       │

├─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│Time to Live   │  Protocol     │       Header Checksum         │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                      Source IP Address                        │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                    Destination IP Address                     │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┤

│                 Options (variable)            |   Padding     │

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘

Internet Protocol 802.3/802.2/SNAP Frame Format

─────────────────────────────────────────────────

(7)    (1)    (6)       (6)    (2)│<────(46-1500 bytes)────>│

┌────────┬───┬─────────┬─────────┬───┬─────────────────────────┬───┐

802.3   │Preamble│SFD│Dest Addr│ Src Addr│Len│        Data     |  Pad  │FCS│

└────────┴───┴─────────┴─────────┴───┴─────────────────────────┴───┘

┌────────────────────────────────────┘                         └───┐

│ (1)  (1)  (1)                                                    │

┌────┬────┬────┬────────┬──────────────────────────────────────────┐

802.2   │DSAP│SSAP│CTRL│SNAP Hdr│        Information                       │

└────┴────┴────┴────────┴──────────────────────────────────────────┘

AA   AA   03 │        │                                          │

┌─────┘        │                                          │

│   (3)    (2) │                                          │

┌────────┬─────┐                                          │

│00│00│00│Type │                                          │

└────────┴─────┘                                          │

0800 │                                          │

┌────────────────────┘                                          │

│0               8              16              24            31│

┌───────┬───────┼───────────────┼───────────────┴───────────────┐

│Version│  IHL  │Type of Service│        Total Length           │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┬─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│        Identification         │flags│   Fragment Offset       │

├─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│Time to Live   │  Protocol     │       Header Checksum         │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                      Source IP Address                        │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                    Destination IP Address                     │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┤

│                 Options (variable)            |   Padding     │

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘

Internet Protocol 802.5/802.2/SNAP Frame Format

─────────────────────────────────────────────────

(1)(1)(1)    (6)       (6)   (0-30)│<────(variable)─────>│ (4)(1)(1)

┌──┬──┬──┬─────────┬─────────┬─────┬─────────────────────┬───┬──┬──┐

802.5   │SD│AC│FC│Dest Addr│ Src Addr│ RIF │       Data          │FCS│ED│FS│

└──┴──┴──┴─────────┴─────────┴─────┴─────────────────────┴───┴──┴──┘

┌──────────────────────────────────┘                     └─────────┐

│ (1)  (1)  (1)                                                    │

┌────┬────┬────┬────────┬──────────────────────────────────────────┐

802.2   │DSAP│SSAP│CTRL│SNAP Hdr│        Information                       │

└────┴────┴────┴────────┴──────────────────────────────────────────┘

AA   AA   03 │        │                                          │

┌─────┘        │                                          │

│   (3)    (2) │                                          │

┌────────┬─────┐                                          │

│00│00│00│Type │                                          │

└────────┴─────┘                                          │

0800 │                                          │

┌────────────────────┘                                          │

│0               8              16              24            31│

┌───────┬───────┼───────────────┼───────────────┴───────────────┐

│Version│  IHL  │Type of Service│        Total Length           │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┬─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│        Identification         │flags│   Fragment Offset       │

├─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│Time to Live   │  Protocol     │       Header Checksum         │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                      Source IP Address                        │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                    Destination IP Address                     │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┤

│                 Options (variable)            |   Padding     │

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘