IP Routing Information Protocol Frame Format

──────────────────────────────────────────────

┌────────────┬───────────┬────────────┬─────────────────┬─────────────┐

│ MAC Header │ IP Header │ UDP Header │ RIP Header/Data │ MAC Trailer │

└────────────┴───────────┴────────────┴─────────────────┴─────────────┘

│                 │

┌──────────────────────────────────┘                 └──────────┐

│0               8              16              24            31│

┌───────────────┼───────────────┼───────────────┴───────────────┐

│  Command(1-5) │ Version (1)   │         Must be zero          │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│      Family of Net 1          │         Must be zero          │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                      IP Address of Net 1                      │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                          Must be zero                         │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                          Must be zero                         │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                       Distance to Net 1                       │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│      Family of Net 2          │         Mut be zero           │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                      IP Address of Net 2                      │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                          Must be zero                         │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                          Must be zero                         │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                       Distance to Net 2                       │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                            etc...                             │

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘

Command: Specifies the operation being performed

1 = Request for partial or full routing information

2 = Response containing sender's routing table

3 = Turn on trace mode (obsolete)

4 = Turn off trace mode (obsolete)

5 = Reserved for Sun Microsystems

Version: Protocol version number, current version is (1).

Family of Net n: Identifies the protocol family of the network address.

IP is assigned a value of (2).

IP Address: The address of the network being advertised.

Distance to net: A count of the number of networks that must be traversed

to reach the network being advertised. Range=1-16, where

16 is an unreachable network.