Novell's Internetwork Packet Exchange (IPX) protocol

──────────────────────────────────────────────────────

IPX is a connectionless, Network layer, datagram protocol.  No formal

connection is required in order to communicate via IPX.  To send an IPX

packet, the station simply addresses it and transmits.  There is no guaranty

that the packet will reach it's destination.

Novell IPX Packet Structure

─────────────────────────────

┌────────────┬─────────────┬─────────────────────────┬─────────────┐

│ MAC Header │ IPX Header  │   Layer 4 Header/Data   │ MAC Trailer │

└────────────┴─────────────┴─────────────────────────┴─────────────┘

│             │                         │

┌────────────┘             └─────────────────┐       └─────────────┐

│(2)  (2)(1) (1)  (4) (6)  (2)  (4) (6)  (2) │<─────(variable)────>│

┌────┬───┬──┬────┬───┬────┬────┬───┬────┬────┬─────────────────────┐

│Cksm│Len│TC│Type│Net│Node│Sckt│Net│Node│Sckt│        Data         │

└────┴───┴──┴────┴───┴────┴────┴───┴────┴────┴─────────────────────┘

└─── Dest ────┴──── Src ────┘

└─────────────────── IPX Length field count ───────────────────────┘

CKSM: Checksum

Not used.  Always = 0xFFFF.

LEN: Length

The length of the IPX packet, including IPX header and Data.

TC: Transport Control

Used by IPX routers to count the number of hops a packet has

made.  The value is initially set to "0" and is incremented

each time the packet crosses a router.  If the value reaches

"16" then the packet is discarded.

Type: Packet Type

One byte Novell packet type.

┌───────┬───────┬─────────────────────────────────────────────────┐

│ (hex) │ (dec) │           IPX Packet Types                      │

├───────┼───────┼─────────────────────────────────────────────────┤

│     0 │     0 │ Unknown packet type                             │

│     1 │     1 │ Routing Information Protocol (RIP) packet       │

│     2 │     2 │ Echo Packet                                     │

│     3 │     3 │ Error Packet                                    │

│     4 │     4 │ Internet Packet Exchange (IPX) protocol packet  │

│     5 │     5 │ Sequenced Packet Exchange (SPX) protocol packet │

│    11 │    17 │ NetWare Core Protocol (NCP)                     │

│    14 │    20 │ IPX NetBIOS                                     │

│ 16-31 │ 22-49 │ Experimental Protocols                          │

└───────┴───────┴─────────────────────────────────────────────────┘

Destination IPX Address

─────────────────────────

NET: Destination IPX Network Number

Four-byte Destination Network Number (0xNNNNNNNN).  If set to

"0x00000000", the destination node is assumed to be on the same

local network network segment as the sending (or source) node.

IPX Routers will not propagate a packet with a Destination

Net = 0x00000000.  Note: There is no broadcast Net Number

(0xFFFFFFFF) allowed in IPX.

NODE: Destination IPX Node Number

Six-byte Destination Node Number (0xNNNNNNNNNNNN).  Set to the

value of the destination station's Ethernet or 802.X address,

except for NetWare 3.X fileserver internal network node numbers

that default to "0x000000000001".  A Destination Node number of

"0xFFFFFFFFFFFF" indicates a broadcast to all nodes on the

destination network.

SCKT: Destination IPX Socket Number

Two-byte Destination Socket Number (0xNNNN).  See list below.

Note: There is no broadcast socket number of "0xFFFF".

Source IPX Address

────────────────────

NET: Source IPX Network Number

Four-byte source network number (0xNNNNNNNN).  If set to

"0x00000000", the source device does not know its network number.

This situation occurs when a workstation is first booted.  If an

IPX Router needs to forward a packet that has a Source Net number

of "0x00000000", it must first replace the Source Net number with

the appropriate Source Net Address.

NODE: Source IPX Node Number

Six-byte Source Node Number (0xNNNNNNNNNNNN).  Set to the value

of the source station's Ethernet or 802.X MAC address, except

for NetWare 3.X fileserver internal network node numbers that

default to "000000000001".

SCKT: Source IPX Socket Number

Two-byte Source Socket Number (0xNNNN).  See list below.  This

is the socket number of the process that generated the packet.

┌─────────────────────────────────────────────────────┐

│               IPX Socket Numbers                    │

├─────────────────────────────────────────────────────┤

│ 0451       NetWare Core Protocol (NCP)              │

│ 0452       Service Advertising Protocol (SAP)       │

│ 0453       Routing Information Protocol (RIP)       │

│ 0455       Novell NetBIOS Packet                    │

│ 0456       Diagnostic Packet                        │

│ 0457       Serialization (Copy Protection) Packet   │

│ 4000-7FFF  Dynamically Assigned by F/S or W/S shell │

│ 8000-FFFF  Well-known Sockets (TL/100's User Added) │               │

└─────────────────────────────────────────────────────┘

DATA: Data field

Contains upper layer protocol information and/or data.

Ethernet / IEEE802.3 / and IEEE802.5 IPX Formats

──────────────────────────────────────────────────

Note: See c:\seminar\files\info\network\ipx\tl100.txt for TL/100 IPX

encapsulation parameter table...

Novell 802.3 "Raw" Frame Format

─────────────────────────────────

(7)    (1)    (6)       (6)    (2)│<────(46-1500 bytes)────>│(4)

┌────────┬───┬─────────┬─────────┬───┬─────┬───────────────────┬───┐

802.3   │Preamble│SFD│Dest Addr│ Src Addr│Len│ IPX │  Data     |  Pad  │FCS│

└────────┴───┴─────────┴─────────┴───┴─────┴───────────────────┴───┘

┌────────────────────────────────────┘     └─┐                 └───┐

│(2)  (2)(1) (1)  (4) (6)  (2)  (4) (6)  (2) │   (16-1470 bytes)   │

┌────┬───┬──┬────┬───┬────┬────┬───┬────┬────┬─────────────────────┐

│Cksm│Len│TC│Type│Net│Host│Sckt│Net│Host│Sckt│        Data         │

└────┴───┴──┴────┴───┴────┴────┴───┴────┴────┴─────────────────────┘

FFFF            └─── Dest ────┴──── Src ────┘

└─────────────────── IPX Length field count ───────────────────────┘

Novell Ethernet II Frame Format

─────────────────────────────────

(8)        (6)       (6)    (2) │<────(46-1500 bytes)────>│(4)

┌───────────┬─────────┬─────────┬────┬─────┬───────────────────┬───┐

Eth II  │ Preamble  │Dest Addr│ Src Addr│Type│ IPX │       Data        │FCS│

└───────────┴─────────┴─────────┴────┴─────┴───────────────────┴───┘

8137│     │                   │

┌────────────────────────────────────┘     └─┐                 └───┐

│(2)  (2)(1) (1)  (4) (6)  (2)  (4) (6)  (2) │   (16-1470 bytes)   │

┌────┬───┬──┬────┬───┬────┬────┬───┬────┬────┬─────────────────────┐

│Cksm│Len│TC│Type│Net│Host│Sckt│Net│Host│Sckt│        Data         │

└────┴───┴──┴────┴───┴────┴────┴───┴────┴────┴─────────────────────┘

FFFF            └─── Dest ────┴──── Src ────┘

└─────────────────── IPX Length field count ───────────────────────┘

Novell 802.3/802.2 Frame Format

─────────────────────────────────

(7)    (1)    (6)       (6)    (2)│<────(46-1500 bytes)────>│(4)

┌────────┬───┬─────────┬─────────┬───┬─────────────────────────┬───┐

802.3   │Preamble│SFD│Dest Addr│ Src Addr│Len│        Data     |  Pad  │FCS│

└────────┴───┴─────────┴─────────┴───┴─────────────────────────┴───┘

┌────────────────────────────────────┘                         └───┐

│ (1)  (1)  (1)  (30)                                              │

┌────┬────┬────┬─────┬─────────────────────────────────────────────┐

802.2   │DSAP│SSAP│CTRL│ IPX │                Information                  │

└────┴────┴────┴─────┴─────────────────────────────────────────────┘

E0   E0   03 │     │                                             │

┌──────────────┘     └───────────────────────┐                     │

│(2)  (2)(1) (1)  (4) (6)  (2)  (4) (6)  (2) │   (13-1467 bytes)   │

┌────┬───┬──┬────┬───┬────┬────┬───┬────┬────┬─────────────────────┐

IPX   │Cksm│Len│TC│Type│Net│Host│Sckt│Net│Host│Sckt│         Data        │

└────┴───┴──┴────┴───┴────┴────┴───┴────┴────┴─────────────────────┘

│FFFF            └─── Dest ────┴──── Src ────┘                     │

└──────────────────── IPX Length field count ──────────────────────┘

Novell 802.3/802.2/SNAP Frame Format

──────────────────────────────────────

(7)    (1)    (6)       (6)    (2)│<────(46-1500 bytes)────>│

┌────────┬───┬─────────┬─────────┬───┬─────────────────────────┬───┐

802.3   │Preamble│SFD│Dest Addr│ Src Addr│Len│        Data     |  Pad  │FCS│

└────────┴───┴─────────┴─────────┴───┴─────────────────────────┴───┘

┌────────────────────────────────────┘                         └───┐

│ (1)  (1)  (1)   (5)     (30)                                     │

┌────┬────┬────┬────────┬─────┬────────────────────────────────────┐

802.2   │DSAP│SSAP│CTRL│SNAP Hdr│ IPX │             Information            │

└────┴────┴────┴────────┴─────┴────────────────────────────────────┘

AA   AA   03 │        │     │                                    │

┌─────┘        │     │                                    │

│   (3)    (2) │     │                                    │

┌────────┬─────┐     │                                    │

│00│00│00│Type │     │                                    │

└────────┴─────┘     │                                    │

8137 │     │                                    │

┌───────────────────────┘     └──────────────┐                     │

│(2)  (2)(1) (1)  (4) (6)  (2)  (4) (6)  (2) │   (8-1462 bytes)    │

┌────┬───┬──┬────┬───┬────┬────┬───┬────┬────┬─────────────────────┐

IPX   │Cksm│Len│TC│Type│Net│Host│Sckt│Net│Host│Sckt│         Data        │

└────┴───┴──┴────┴───┴────┴────┴───┴────┴────┴─────────────────────┘

│FFFF            └─── Dest ────┴──── Src ────┘                     │

└──────────────────── IPX Length field count ──────────────────────┘

Novell 802.5/802.2 Frame Format

─────────────────────────────────

(1)(1)(1)    (6)       (6)   (0-30)│<────(variable)─────>│ (4)(1)(1)

┌──┬──┬──┬─────────┬─────────┬─────┬─────────────────────┬───┬──┬──┐

802.5   │SD│AC│FC│Dest Addr│ Src Addr│ RIF │       Data          │FCS│ED│FS│

└──┴──┴──┴─────────┴─────────┴─────┴─────────────────────┴───┴──┴──┘

┌──────────────────────────────────┘                     └─────────┐

│ (1)  (1)  (1)  (30)                                              │

┌────┬────┬────┬─────┬─────────────────────────────────────────────┐

802.2   │DSAP│SSAP│CTRL│ IPX │           Information                       │

└────┴────┴────┴─────┴─────────────────────────────────────────────┘

E0   E0   03 │     │                                             │

┌──────────────┘     └───────────────────────┐                     │

│(2)  (2)(1) (1)  (4) (6)  (2)  (4) (6)  (2) │<────(variable)─────>│

┌────┬───┬──┬────┬───┬────┬────┬───┬────┬────┬─────────────────────┐

IPX   │Cksm│Len│TC│Type│Net│Host│Sckt│Net│Host│Sckt│         Data        │

└────┴───┴──┴────┴───┴────┴────┴───┴────┴────┴─────────────────────┘

│FFFF            └─── Dest ────┴──── Src ────┘                     │

└──────────────────── IPX Length field count ──────────────────────┘

Novell 802.5/802.2/SNAP Frame Format

──────────────────────────────────────

(1)(1)(1)    (6)       (6)   (0-30)│<────(variable)─────>│ (4)(1)(1)

┌──┬──┬──┬─────────┬─────────┬─────┬─────────────────────┬───┬──┬──┐

802.5   │SD│AC│FC│Dest Addr│ Src Addr│ RIF │       Data          │FCS│ED│FS│

└──┴──┴──┴─────────┴─────────┴─────┴─────────────────────┴───┴──┴──┘

┌──────────────────────────────────┘                     └─────────┐

│ (1)  (1)  (1)   (5)     (30)                                     │

┌────┬────┬────┬────────┬─────┬────────────────────────────────────┐

802.2   │DSAP│SSAP│CTRL│SNAP Hdr│ IPX │            Information             │

└────┴────┴────┴────────┴─────┴────────────────────────────────────┘

AA   AA   03 │        │     │                                    │

┌─────┘        │     │                                    │

│   (3)    (2) │     │                                    │

┌────────┬─────┐     │                                    │

│00│00│00│Type │     │                                    │

└────────┴─────┘     │                                    │

8137 │     │                                    │

┌───────────────────────┘     └──────────────┐                     │

│(2)  (2)(1) (1)  (4) (6)  (2)  (4) (6)  (2) │<─────(variable)────>│

┌────┬───┬──┬────┬───┬────┬────┬───┬────┬────┬─────────────────────┐

IPX   │Cksm│Len│TC│Type│Net│Host│Sckt│Net│Host│Sckt│         Data        │

└────┴───┴──┴────┴───┴────┴────┴───┴────┴────┴─────────────────────┘

│FFFF            └─── Dest ────┴──── Src ────┘                     │

└──────────────────── IPX Length field count ──────────────────────┘