MAC Addresses

───────────────

MAC addresses consist of a 48 bit binary address.  The first 24 bits of the

address are assigned by the IEEE an comprise the vendor's ID (See FILES\INFO\

\DATALINK\ETH_VEND.TXT for a list of vendor IDs in Ethernet Format).  The last

24 bits are a unique serial number for the network interface card that is

assigned by the vendor.  The 24 bit vendor ID for Timeplex is:

0001 0000 0000 0000 1101 0000

This value, along with the 24 bit serial number of the NIC, is stored on the

NIC card in memory, usually in a PROM.  For example, let's assume that

Timeplex assigned a serial number like this to a certain NIC card:

0100 1000 0010 1100 0110 1010

The complete MAC address would be:

0001 0000 0000 0000 1101 0000 0100 1000 0010 1100 0110 1010

The address would be stored, byte by byte in memory like this:

7654 3210   <─── bit ordering

-----------

0001 0000   <─── first byte of address

0000 0000

1101 0000

0100 1000

0010 1100

0110 1010   <─── last byte of address

An ethernet or 802.3 NIC will transmit data out of it's interface Low order

bit first and an 802.5 or an FDDI interface will transmit data High order bit

first.  For this reason, the vendor's ID appears differently on the LANalyzer

for token ring and ethernet.

For Token Ring or FDDI, the address above can be easily converted to hex:

binary:   0001 0000│0000 0000│1101 0000│0100 1000│0010 1100│0110 1010

hex:    1    0  │ 0    0  │ D    0  │ 4    8  │ 2    C  │ 6    A

For ethernet, the bit order of the individual bytes in the address must be

reversed before being converted to hex:

original:   0001 0000│0000 0000│1101 0000│0100 1000│0010 1100│0110 1010

reversed:   0000 1000│0000 0000│0000 1011│0001 0010│0011 0100│0101 0110

hex:    0    8  │ 0    0  │ 0    B  │ 1    2  │ 3    4  │ 5    6

Now we see the familiar "08 00 0B xx xx xx" style Timeplex MAC address.