Token Ring MAC Frame formats

─────────────────────────────

┌──────────── Code Violation Protected ────────────────┐

│  ┌─────────────── FCS Protected ─────────────────────┐

(1)(1)(1)    (6)       (6)             (variable)          (4)(1)(1)

┌──┬──┬──┬─────────┬─────────┬────────────────────────────┬───┬──┬──┐

│SD│AC│FC│Dest Addr│ Src Addr│     Information Field      │FCS│ED│FS│

└──┴──┴──┴─────────┴─────────┴────────────────────────────┴───┴──┴──┘

┌────────────────────────────┘                            └─────────┐

│ (2) (1) (1 or 3)                      (var)                       │

┌─────┬──┬──┬──┬──┬─────────────────────────────────────────────────┐

│ Len │CL│  CMD   │                 Subvectors                      │

└─────┴──┴──┴──┴──┴─────────────────────────────────────────────────┘

└── MV ID ──┘                                                 │

┌─────────────────┘                                                 │

│                                                                   │

┌─────────────┬─────────────┬─────────────────────────┬─────────────┐

│ Subvector 1 │ Subvector 2 │          ...            │ Subvector n │

└─────────────┴─────────────┴─────────────────────────┴─────────────┘

│             └─────────────┐

│(1) (1)        (var)       │

┌───┬────┬──────────────────┐

│Len│Type│      Value       │

└───┴────┴──────────────────┘

MAC Frame Field Descriptions

──────────────────────────────

┌─┬─┬─┬─┬─┬─┬─┬─┐

AC: │F│F│0│0│Z│Z│Z│Z│

└─┴─┴─┴─┴─┴─┴─┴─┘

FF = 00 = MAC frame

ZZZZ = Physical Control Field (PCF) Attention Code

┌──────┬─────────────────────────┐

│ PCF  │ MAC frame type          │

├──────┼─────────────────────────┤

│ 0000 │ Normal Buffer           │

│ 0001 │ Express Buffer          │

│ 0010 │ Beacon                  │

│ 0011 │ Claim Token             │

│ 0100 │ Ring Purge              │

│ 0101 │ Active Monitor Present  │

│ 0110 │ Standby Monitor Present │

└──────┴─────────────────────────┘

┌─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┐

Len: │  Major Vector │ Length Field  │

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘

Two byte Length field.  Length includes entire Major Vector (MV)

field including Len field and all Subvector fields.

┌─┬─┬─┬─┬─┬─┬─┬─┐

CL: │d│d│d│d│s│s│s│s│ Major Vector (MV) Class

└─┴─┴─┴─┴─┴─┴─┴─┘

dddd = Destination Station MV Class

ssss = Source Station MV Class

┌─────┬────────────────────────────┐

│Class│ SRC/DST Station Description│

├─────┼────────────────────────────┤

│  0  │ Ring Station               │

│  1  │ LLC Manager                │

│  4  │ Network Manager            │

│  5  │ Ring Parameter Server      │

│  6  │ Ring Error Monitor         │

└─────┴────────────────────────────┘

┌─┬─┬─┬─┬─┬─┬─┬─┬───────────────┬───────────────┐

CMD: │  MV Command   │Extended MV CMD field if CMD=FF│

└─┴─┴─┴─┴─┴─┴─┴─┴───────────────┴───────────────┘

┌────┬─────────────────────────────────────┐

│CMD │ MAC Frame Major Vector              │

├────┼─────────────────────────────────────┤

│ 00 │ Response                            │

│ 02 │ Beacon                              │

│ 03 │ Claim Token                         │

│ 04 │ Ring Purge                          │

│ 05 │ Acive Monitor Present (AMP)         │

│ 06 │ Standby Monitor Present (SMP)       │

│ 07 │ Duplicate Address Found             │

│ 08 │ Lobe Media Test                     │

│ 09 │ Transmit Forward                    │

│ 0B │ Remove Ring Station                 │

│ 0C │ Change Parameters                   │

│ 0D │ Initialize Ring Station             │

│ 0E │ Request Station Address             │

│ 0F │ Request Station State               │

│ 10 │ Request Station Attach              │

│ 20 │ Request Initialization              │

│ 22 │ Report Station Address              │

│ 23 │ Report Station State                │

│ 24 │ Report Station Attach               │

│ 25 │ Report New Monitor                  │

│ 26 │ Report SUA Change                   │

│ 27 │ Report Poll Failure                 │

│ 28 │ Report Monitor Error                │

│ 29 │ Report Error                        │

│ 2A │ Report Transmit Forward             │

│ FF │ Extended MV Command field (3 bytes) │

└────┴─────────────────────────────────────┘

┌─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┬─┐─ ─ ─ ┐

SV: │   SV Length   │    SV Type    │    Subvector Value Field (variable)  |

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘─ ─ ─ ┘

MAC frame Subvector Types:

────────────────────────────

╔════╪════╪════════════════════════════════╪═════════════════════════════════╗

║Len │Type│ Subvector Value (var len)      │ SV Type/Value field description ║

╠════╪════╪═════════╪══════════════════════╪═════════════════════════════════╣

║ 04 │ 01 │ XX | XX │                      │ Beacon Type                     ║

║    │    │ 00 | 01 │                      │ -Set Recovery Mode              ║

║    │    │ 00 | 02 │                      │ -Ring Signal Loss               ║

║    │    │ -- | -- │                      │  Monitor Contention Failed      ║

║    │    │ 00 | 03 │                      │  -no contention frames          ║

║    │    │ 00 | 04 │                      │  -contention frame received     ║

╠════╪════╪═════════╧═══════════════════╕  ╞═════════════════════════════════╣

║ 08 │ 02 │ XX | XX | XX | XX | XX | XX │  │ Upstream Neighbor's Address UNA ║

╠════╪════╪═════════╪═══════════════════╛  ╞═════════════════════════════════╣

║ 04 │ 03 │ XX | XX │                      │ Local Ring Number               ║

╠════╪════╪═════════╧═════════╕            ╞═════════════════════════════════╣

║ 06 │ 04 │ XX | XX | XX | XX │            │ Assign Physical Drop Number     ║

╠════╪════╪═════════╪═════════╛            ╞═════════════════════════════════╣

║ 04 │ 05 │ XX | XX │                      │ Soft Error Report Timer (0-ffff)║

╠════╪════╪═════════╡                      ╞═════════════════════════════════╣

║ 04 │ 06 │ XX | XX │                      │ Enabled Function Classes (0-15) ║

╠════╪════╪═════════╡                      ╞═════════════════════════════════╣

║ 04 │ 07 │ XX | XX │                      │ Allowed access priority (0-3)   ║

╠════╪════╪═════════╡                      ╞═════════════════════════════════╣

║ 04 │ 08 │ XX | XX │                      │ Authorized Environment          ║

╠════╪════╪═════════╡                      ╞═════════════════════════════════╣

║ 04 │ 09 │ XX | XX │                      │ Correlator (to relate frames)   ║

╠════╪════╪═════════╧═══════════════════╕  ╞═════════════════════════════════╣

║ 08 │ 0A │ XX | XX | XX | XX | XX | XX │  │ Last AMP/SMP Addr,ring poll fail║

╠════╪════╪═══════════════════╪═════════╛  ╞═════════════════════════════════╣

║ 06 │ 0B │ XX | XX | XX | XX │            │ Physical Drop Number            ║

╠════╪════╪═══════════════════╡            ╞═════════════════════════════════╣

║ 06 │ 20 │ XX | XX | CL | CMD│            │ Response Code                   ║

║    │    │ 00 | 01 |         │            │ - ACK                           ║

║    │    │ 80 | 02 | Class & │            │ - MAC frame data fld incomplete ║

║    │    │ 80 | 03 | Major   │            │ - MV length invalid             ║

║    │    │ 80 | 04 | Vector  │            │ - Inappropriate source class    ║

║    │    │ 80 | 05 | Command │            │ - SV length invalid             ║

║    │    │ 80 | 06 | from the│            │ - Transmit forward frame error  ║

║    │    │ 80 | 07 | MAC     │            │ - SV missing                    ║

║    │    │ 80 | 08 | frame   │            │ - SV unknown                    ║

║    │    │ 80 | 09 | MV field│            │ - MAC frame exceeds max length  ║

║    │    │ 80 | 0A |         │            │ - Function disabled             ║

╠════╪════╪═════════╪═════════╛            ╞═════════════════════════════════╣

║ 04 │ 21 │ 00 | 00 │                      │ Reserved                        ║

╠════╪════╪═════════╧═══════════════════╕  ╞═════════════════════════════════╣

║ 14 │ 22 │ XX | XX | XX | XX | XX | XX │  │ Product Instance ID             ║

║    │    ├─────────────────────────────┤  │  (not used by adapter)          ║

║    │    │ XX | XX | XX | XX | XX | XX │  │                                 ║

║    │    ├─────────────────────────────┤  │                                 ║

║    │    │ XX | XX | XX | XX | XX | XX │  │                                 ║

╠════╪════╪════════════════════════╪════╛  ╞═════════════════════════════════╣

║    │    │ FC | FC | FC | FC | FC │       │ Adapter Software Level          ║

║ 0C │ 23 ├────────────────────────┤       │  FC = Feature Code              ║

║    │    │ EC | EC | EC | EC | EC │       │  EC = EC Level                  ║

╠════╪════╪════════════════════════╧═════╕ ╞═════════════════════════════════╣

║ vv │ 26 │         variable             | │ Wrap Data                       ║

╠════╪════╪══════════════════════════════╡ ╞═════════════════════════════════╣

║ vv │ 27 │         variable             | │ Frame to Forward (ACbyte──>Info)║

╠════╪════╪═════════════════════════════╪╛ ╞═════════════════════════════════╣

║ 08 │ 29 │ 0  | 1  | 2  | 3  | 4  | 5  │  │ Adapter Status Word             ║

║    │    ╞═════════════════════════════╛  └─────────────────────────────────╢

║    │    │                                                                  ║

║    │    │   Status Info Field Task Word       SSB = System Status Block    ║

║    │    │     byte 0          byte 1                                       ║

║    │    │ ├─┬─┬─┬─┬─┬─┬─┬─┼─┬─┬─┬─┬─┬─┬─┬─┤                                ║

║    │    │ │0│1│2│3│4│5│6│7│0│1│2│3│4│5│6│7│                                ║

║    │    │ └─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘                                ║

║    │    │  │ │ │ │   │ │   │ │ │ │ │                                       ║

║    │    │  │ │ │ │   │ │   │ │ │ │ └───────── SSB Wait Transmit            ║

║    │    │  │ │ │ │   │ │   │ │ │ └─────────── SSB Wait Receive             ║

║    │    │  │ │ │ │   │ │   │ │ └───────────── SSB Wait Command Comp        ║

║    │    │  │ │ │ │   │ │   │ └─────────────── SSB Wait Ring Status         ║

║    │    │  │ │ │ │   │ │   └───────────────── SSB Wait SCB Clear           ║

║    │    │  │ │ │ │   │ └───────────────────── Transmit Command             ║

║    │    │  │ │ │ │   └─────────────────────── Receive Command              ║

║    │    │  │ │ │ └─────────────────────────── Adapter Open                 ║

║    │    │  │ │ └───────────────────────────── SSB Queued                   ║

║    │    │  │ └─────────────────────────────── SSB Busy                     ║

║    │    │  └───────────────────────────────── SIF Initialized              ║

║    │    │                                                                  ║

║    │    │     Transmit Status Word                                         ║

║    │    │     byte 2          byte 3                                       ║

║    │    │ ├─┬─┬─┬─┬─┬─┬─┬─┼─┬─┬─┬─┬─┬─┬─┬─┤                                ║

║    │    │ │0│1│2│3│4│5│6│7│0│1│2│3│4│5│6│7│                                ║

║    │    │ └─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘                                ║

║    │    │  │ │ │   │ │ │   │   │ │                                         ║

║    │    │  │ │ │   │ │ │   │   │ └─────────── CSTAT EOF Bit                ║

║    │    │  │ │ │   │ │ │   │   └───────────── CSTAT SOF Bit                ║

║    │    │  │ │ │   │ │ │   └───────────────── CSTAT Valid Bit              ║

║    │    │  │ │ │   │ │ └───────────────────── List Error                   ║

║    │    │  │ │ │   │ └─────────────────────── Expecting SOF List           ║

║    │    │  │ │ │   └───────────────────────── Transmit Halt Enabled        ║

║    │    │  │ │ └───────────────────────────── Wait Validation              ║

║    │    │  │ └─────────────────────────────── Wait Transmit Complete       ║

║    │    │  └───────────────────────────────── Subtask Not Ready            ║

║    │    │                                                                  ║

║    │    │          Receive Status                                          ║

║    │    │     byte 4          byte 5                                       ║

║    │    │ ├─┬─┬─┬─┬─┬─┬─┬─┼─┬─┬─┬─┬─┬─┬─┬─┤                                ║

║    │    │ │0│1│2│3│4│5│6│7│0│1│2│3│4│5│6│7│                                ║

║    │    │ └─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘                                ║

║    │    │  │ │ │           │ │                                             ║

║    │    │  │ │ │           │ └─────────────── RI-pad Escape                ║

║    │    │  │ │ │           └───────────────── Expecting SOF List           ║

║    │    │  │ │ └───────────────────────────── Waiting for Validation       ║

║    │    │  │ └─────────────────────────────── Waiting for Continue         ║

║    │    │  └───────────────────────────────── Subtask Not Ready            ║

║    │    │                                                                  ║

╠════╪════╪═════════╪══════════════════════╪═════════════════════════════════╣

║ 04 │ 2A │ XX | XX │                      │ Transmit Status Code            ║

╠════╪════╪═════════╧═════════╕            ╞═════════════════════════════════╣

║ 06 │ 2B │ XX | XX | XX | XX │            │ Group Address                   ║

╠════╪════╪═══════════════════╡            ╞═════════════════════════════════╣

║ 06 │ 2C │ XX | XX | XX | XX │            │ Functional Address              ║

╠════╪════╪═══════════════════╧═════════╕  ╞═════════════════════════════════╣

║ 08 │ 2D │ 0  | 1  | 2  | 3  | 4  | 5  │  │ Isolating Error Counts          ║

║    │    ╞═════════════════════════════╛  │ ┌────┬───────────────┐          ║

║    │    │                                │ │Byte│  Error Count  │          ║

║    │    │                                │ ├────┼───────────────┤          ║

║    │    │                                │ │ 0  │ Line Error    │          ║

║    │    │                                │ │ 1  │ reserved      │          ║

║    │    │                                │ │ 2  │ Burst Error   │          ║

║    │    │                                │ │ 3  │ ARI/FCI Error │          ║

║    │    │                                │ │ 4  │ reserved      │          ║

║    │    │                                │ │ 5  │ reserved      │          ║

║    │    │                                │ └────┴───────────────┘          ║

╠════╪════╪═════════════════════════════╪══╪═════════════════════════════════╣

║ 08 │ 2E │ 0  | 1  | 2  | 3  | 4  | 5  │  │ Non-Isolating Error Counts      ║

║    │    ╞═════════════════════════════╛  │ ┌────┬────────────────────┐     ║

║    │    │                                │ │Byte│  Error Count       │     ║

║    │    │                                │ ├────┼────────────────────┤     ║

║    │    │                                │ │ 0  │ Lost Frame         │     ║

║    │    │                                │ │ 1  │ Receive Congestion │     ║

║    │    │                                │ │ 2  │ Frame Copied Error │     ║

║    │    │                                │ │ 3  │ reserved           │     ║

║    │    │                                │ │ 4  │ Token Error        │     ║

║    │    │                                │ │ 5  │ reserved           │     ║

║    │    │                                │ └────┴────────────────────┘     ║

╠════╪════╪═════════════════════════════╕  ╞═════════════════════════════════╣

║ 08 │ 2F │ XX | XX | XX | XX | XX | XX │  │ Function Request ID             ║

╠════╪════╪═════════╪═══════════════════╛  ╞═════════════════════════════════╣

║ 04 │ 30 │ XX | XX │                      │ Error Code                      ║

║    │    │ 00 | 01 │                      │ - Monitor Error                 ║

║    │    │ 00 | 02 │                      │ - Duplicate Monitor             ║

║    │    │ 00 | 03 │                      │ - Duplicate Address             ║

╠════╪════╪═════════╧══════════════════════╪═════════════════════════════════╣

║ FF │    │                                │ Extended Length Field (2 bytes) ║

╠════╪════╪════════════════════════════════╪═════════════════════════════════╣

║    │ FF │                                │ Extended Type Field (2 bytes)   ║

╚════╧════╧════════════════════════════════╧═════════════════════════════════╝