Ethernet Multicast & Broadcast Addresses

──────────────────────────────────────────

Multicast Address    EtherType       Description/Usage

───────────────────   ─────────  ─────────────────────────────

01-00-5E-00-00-00       0800    Internet Multicast (RFC-1112)

01-00-5E-7F-FF-FF

01-00-5E-80-00-00       ????    Internet reserved by IANA

01-00-5E-FF-FF-FF

* 01-00-B0-00-10-00       NONE    Timeplex TL/10 Multicast Address

* 01-80-C2-00-00-00       -802-   802.1d Bridge Group Address (STA/P)

01-80-C2-00-00-01               Reserved for future use

01-80-C2-00-00-02               Reserved for future use

01-80-C2-00-00-03               Reserved for future use

01-80-C2-00-00-04               Reserved for future use

01-80-C2-00-00-05               Reserved for future use

01-80-C2-00-00-06               Reserved for future use

01-80-C2-00-00-07               Reserved for future use

01-80-C2-00-00-08               Reserved for future use

01-80-C2-00-00-09               Reserved for future use

01-80-C2-00-00-0A               Reserved for future use

01-80-C2-00-00-0B               Reserved for future use

01-80-C2-00-00-0C               Reserved for future use

01-80-C2-00-00-0D               Reserved for future use

01-80-C2-00-00-0E               Reserved for future use

01-80-C2-00-00-0F               Reserved for future use

* 01-80-C2-00-00-10       -802-   802.1d Bridge Management Group Address

09-00-02-04-00-01?      8080?   Vitalink printer

09-00-02-04-00-02?      8080?   Vitalinnk management

09-00-09-00-00-01       8005    HP Probe

09-00-09-00-00-01       -802-   HP Probe

09-00-09-00-00-04       8005?   HP DTC

09-00-1E-00-00-00       8019?   Apollo DOMAIN

09-00-2B-00-00-00       6009?   DEC MUMPS?

09-00-2B-00-00-01       8039?   DEC DSM/DTP?

09-00-2B-00-00-02       803B?   DEC VAXELN?

09-00-2B-00-00-03       8038    DEC LANbridge traffic monitor

09-00-2B-00-00-04       ????    DEC MAP End System Hello?

09-00-2B-00-00-05       ????    DEC MAP Intermediate System Hello?

09-00-2B-00-00-06       803D?   DEC CSMA/CD Encryption

09-00-2B-00-00-07       8040?   DEC NetBIOS Emulator?

09-00-2B-00-00-0F       6004    DEC Local Area Transport (LAT)

09-00-2B-00-00-1x       ????    DEC Experimental

09-00-2B-01-00-00       8038    DEC LANBridge Copy Packets

* 09-00-2B-01-00-01       8038    DEC LANBridge Hello Packets

09-00-2B-02-00-00       ????    DEC DNA Level 2 Routing Layer?

09-00-2B-02-01-00       803C?   DEC DNA Naming Service Advertisement?

09-00-2B-02-01-01       803C?   DEC DNA Naming Service Solicitation?

09-00-2B-02-01-02       803E?   DEC DNA Time Service?

09-00-2B-03-xx-xx       ????    DEC default filtering by bridges?

09-00-2B-04-00-00       8041?   DEC Local Area System Transport (LAST)?

09-00-2B-23-00-00       803A?   DEC Argonaut Console?

* 09-00-4E-00-00-02?      8137    Novell IPX

09-00-56-00-00-00       ????    Stanford reserved

09-00-56-FE-FF-FF       ????    unknown

09-00-56-FF-00-00       805C    Stanford V kernel, version 6.0

09-00-56-FF-FF-FF       ????    unknown

09-00-77-00-00-01       ????    Retix Spanning Tree Bridges

09-00-7C-02-00-05       8080?   Vitalink Diagnostics

09-00-7C-05-00-01       8080?   Vitalink Gateway?

0D-1E-15-BA-DD-06       ????    HP

AB-00-00-01-00-00       6001    DEC Maint Operation Protocol (MOP)

dump/load assistance

AB-00-00-02-00-00       6002    DEC Maint Operation Protocol (MOP)

System ID packets from DEC LANbridge

every 8-10 minutes.

AB-00-00-03-00-00       6003    DECNET Phase IV end node Hello packets.

One packet every 15 seconds sent by

each DECNET host.

AB-00-00-04-00-00       6003    DECNET Phase IV Router Hello packets.

One packet every 15 seconds sent by

each DECNET router.

AB-00-00-05-00-00       ????    DEC reserved...

AB-00-03-FF-FF-FF       ????                   ...inclusive.

AB-00-03-00-00-00       6004    DEC Local Area Transport (LAT) - old

AB-00-04-00-xx-xx       ????    Reserved DEC customer private use

AB-00-04-01-xx-xx       6007    DEC Local Area VAX Cluster groups

System Communication Architecture

CF-00-00-00-00-00       9000    Ethernet configuration test protocol

(loopback)

Broadcast Address    EtherType       Description/Usage

───────────────────   ─────────  ────────────────────────────

FF-FF-FF-FF-FF-FF       0600    XNS Hello Packets, 6 packets every

15 seconds per each XNS station

FF-FF-FF-FF-FF-FF       0800    IP (RWHOD, RIP, etc)

FF-FF-FF-FF-FF-FF       0804    CHAOS

FF-FF-FF-FF-FF-FF       0806    ARP (for IP and CHAOS) as needed

FF-FF-FF-FF-FF-FF       0BAD    Banyan

FF-FF-FF-FF-FF-FF       1600    Valid Hello packets, 1 packet every

30 seconds per each VALID station

FF-FF-FF-FF-FF-FF       8035    RARP (Reverse ARP)

FF-FF-FF-FF-FF-FF       807C    Merit Internodal (INP)

FF-FF-FF-FF-FF-FF       809B    EtherTalk (AppleTalk over Ethernet)