Novell NetWare Core Protocol Packet Structures

────────────────────────────────────────────────

NCP Request/Reply Packet Types

┌──────────────────────────────────────┐

│ 0x1111  Create Service Connection    │

│ 0x2222  File Server (NCP) Request    │

│ 0x3333  File Server (NCP) Reply      │

│ 0x5555  Destroy Service Connection   │

│ 0x7777  Burst Mode Request/Reply     │

│ 0x9999  Request Being Processed      │

└──────────────────────────────────────┘

NCP Create Service Connection (0x1111) Packet

───────────────────────────────────────────────

This packet is used by the client workstation to establish a service

connection with the NetWare server.

┌─────────── IPX Length Field ───────────┐

│    (30)                                │

┌────────────┬────────────┬───────────────────────────┬─────────────┐

│ MAC Header │ IPX Header │ NCP Header/Rqst-Rply/Data │ MAC Trailer │

└────────────┴────────────┴───────────────────────────┴─────────────┘

│                           │

┌─┘                           └─┐

│                               │

│ 7   6   5   4   3   2   1   0 │

┌───┬───┬───┬───┬───┬───┬───┬───┐ 0

│    NCP Request type (0x1111)  │

├ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ │

│              ...              │

├───┬───┬───┬───┬───┬───┬───┬───┤ 2

│      Sequence Number (0)      │

├───┬───┬───┬───┬───┬───┬───┬───┤ 3

│    Connection Number (255)    │

└───────────────────────────────┘

NCP Request Type: 0x1111 - NCP Create Service Connection (request)

Sequence Number: Set to zero if the workstation is logging in for the

first time since being initialized (powered-up).

Connection Number: The file server will assign a connection number to

the client.  Since this is a request, and a connection

number has not yet been assigned, this value is set to

255 (0xFF).

NCP File Server Request (0x2222) Packet

───────────────────────────────────────

This packet is used by the client workstation to request that the file server

perform some function.

┌─────────── IPX Length Field ───────────┐

│    (30)                                │

┌────────────┬────────────┬───────────────────────────┬─────────────┐

│ MAC Header │ IPX Header │ NCP Header/Rqst-Rply/Data │ MAC Trailer │

└────────────┴────────────┴───────────────────────────┴─────────────┘

│                           │

┌─┘                           └─┐

│                               │

│ 7   6   5   4   3   2   1   0 │

┌───┬───┬───┬───┬───┬───┬───┬───┐ 0   (bytes)

│    NCP Request type (0x2222)  │

├ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ │

│              ...              │

├───┬───┬───┬───┬───┬───┬───┬───┤ 2

│        Sequence Number        │

├───┬───┬───┬───┬───┬───┬───┬───┤ 3

│     Connection Number Low     │

├───┬───┬───┬───┬───┬───┬───┬───┤ 4

│         Task Number           │

├───┬───┬───┬───┬───┬───┬───┬───┤ 5

│     Connection Number High    │

├───┬───┬───┬───┬───┬───┬───┬───┤ 6

│         Function Code         │

├───┬───┬───┬───┬───┬───┬───┬───┤ 7

│ Subfunction Length (optional) │

├───┬───┬───┬───┬───┬───┬───┬───┤ 8

|  Subfunction Code (optional)  |

|      (variable length)        |

├───┬───┬───┬───┬───┬───┬───┬───┤

|  NCP Request Data (optional)  |

|      (variable length)        |

└───────────────────────────────┘

NCP Request Type: 0x2222 - File Server (NCP) Request

Sequence Number: The sequence number is incremented by one for each

Request - Reply sequence that occurs between the server

and the client.  Note that a request and its associated

reply will have the same sequence number.

Connection Number Low: The low order byte of the connection number assigned

to this logical connection by the file server.

Multiple logical connections may be established

between a file server (or servers) and a client.

The connection number allows the connection to be

uniquely identified.

Task Number: The task number is incremented by one for each task

within a server - client sequence.  For example, a

request would be task number 0 and the subsequent

reply would be task number 1.  The task number begins

at zero for each new function requested.

Connection Number High: The high order byte of the connection number.

See Connection Number Low above.  Note that the high

order byte is not generally used and will usually be

set to 0x00.

Function Code: The function code specifies what function the client is

requesting the file server to perform.

See NCP_NUMS.TXT for a list of function codes.

Subfunction Length: (optional) The length, in bytes, of the subfunction

code that follows.

Subfunction Code: (optional) Certain Function Codes have Subfunction

Codes associated with them.  See NCP_NUMS.TXT for

a list of subfunction codes.

NCP Request Data: (optional) This field contains data that is necessary

for certain NCP Requests (i.e. file name, file handle,

or other information).  If the subfunction fields

above are not used, then this field immediately

follows the function code field.

NCP File Server Reply (0x3333) Packet

───────────────────────────────────────

This packet is used by the file server to reply to client workstation

requests, including type 0x1111 Create Service Connection requests and

type 0x2222 File Server (NCP) Requests.

┌─────────── IPX Length Field ───────────┐

│    (30)                                │

┌────────────┬────────────┬───────────────────────────┬─────────────┐

│ MAC Header │ IPX Header │ NCP Header/Rqst-Rply/Data │ MAC Trailer │

└────────────┴────────────┴───────────────────────────┴─────────────┘

│                           │

┌─┘                           └─┐

│                               │

│ 7   6   5   4   3   2   1   0 │

┌───┬───┬───┬───┬───┬───┬───┬───┐ 0

│     NCP Reply type (0x3333)   │

├ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ │

│              ...              │

├───┬───┬───┬───┬───┬───┬───┬───┤ 2

│        Sequence Number        │

├───┬───┬───┬───┬───┬───┬───┬───┤ 3

│     Connection Number Low     │

├───┬───┬───┬───┬───┬───┬───┬───┤ 4

│         Task Number           │

├───┬───┬───┬───┬───┬───┬───┬───┤ 5

│     Connection Number High    │

├───┬───┬───┬───┬───┬───┬───┬───┤ 6

│        Completion Code        │

├───┬───┬───┬───┬───┬───┬───┬───┤ 7

│       Connection Status       │

├───┬───┬───┬───┬───┬───┬───┬───┤ 8

|   NCP Reply Data (optional)   |

|      (variable length)        |

└───────────────────────────────┘

NCP Reply Type: 0x3333 - File Server (NCP) Reply

Sequence Number: The sequence number is incremented by one for each

Request - Reply sequence that occurs between the server

and the client.  Note that a request and its associated

reply will have the same sequence number.

Connection Number Low: The low order byte of the connection number assigned to

this logical connection by the file server.  Multiple

logical connections may be established between a file

server (or servers) and a client.  The connection

number allows the connection to be uniquely identified.

Task Number: The task number is incremented by one for each task

within a server - client sequence.  For example, a

request would be task number 0 and the subsequent

reply would be task number 1.  The task number begins

at zero for each new function requested.

Connection Number High: The high order byte of the connection number.

See Connection Number Low above.

Completion Code: The status of the requested function.

┌──────┬─────────────────────────────────────────────────────────────┐

│ Code │  Status - Description                                       │

├──────┼─────────────────────────────────────────────────────────────┤

│    0 │ Success - The requested function was completed successfully.│

│  255 │ Failure - The requested function failed.                    │

└──────┴─────────────────────────────────────────────────────────────┘

Connection Status: The logical connection status.  Usually set to 0x00.

NCP Reply Data: (optional) This field contains NCP Reply Function Data.

For example, the data returned by a file read reply

would be contained in this field.

NCP Destroy Service Connection (0x5555) Packet

────────────────────────────────────────────────

This packet is used by the client workstation to destroy a service connection

with the NetWare server.

┌─────────── IPX Length Field ───────────┐

│    (30)                                │

┌────────────┬────────────┬───────────────────────────┬─────────────┐

│ MAC Header │ IPX Header │ NCP Header/Rqst-Rply/Data │ MAC Trailer │

└────────────┴────────────┴───────────────────────────┴─────────────┘

│                           │

┌─┘                           └─┐

│                               │

│ 7   6   5   4   3   2   1   0 │

┌───┬───┬───┬───┬───┬───┬───┬───┐ 0

│    NCP Request type (0x5555)  │

├ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ ┬ ─ │

│              ...              │

├───┬───┬───┬───┬───┬───┬───┬───┤ 2

│        Sequence Number        │

├───┬───┬───┬───┬───┬───┬───┬───┤ 3

│       Connection Number       │

└───────────────────────────────┘

NCP Request Type: 0x5555 - NCP Destroy Service Connection (request)

Sequence Number: The sequence number is incremented by one for each

Request - Reply sequence that occurs between the server

and the client.  Note that a request and its associated

reply will have the same sequence number.

Connection Number: The connection number of the logical connection that the

client workstation wants to terminate.

NCP Burst Mode Request/Reply (0x7777) Packet

──────────────────────────────────────────────

See the file PBURST.TXT for a description of the packet burst protocol.

NCP Request Being Processed (0x9999) Packet

─────────────────────────────────────────────

If a server has a client's request queued for execution, and the client

reexecutes the request because it has not received a reply from the server,

the server responds with a Request Being Processed reply.

A description of the packet structure was not available at the time of

this writing.