Useful Novell NetWare Commands

────────────────────────────────

WORKSTATION COMMANDS AND UTILITIES:

COMCHECK              Utility that tests communication between W/S's.

Comcheck does not require a fileserver to be on the

network.  It only requires IPX.COM be loaded (NETX.COM

is not required).  Run Comcheck on multiple W/S's and

each W/S will display a list of the other stations it

is able to communicate with.  Each station  broadcasts

packets advertising it's presence.  These packets will

pass through IPX routers.

(See Traces\Ethernet\IPX\Comcheck.TXT for packet format)

ECONFIG               Configures IPX.COM for Ethernet II frame format with an

EtherType of 8137 instead of Novell's modified 802.3

format (802.3 without an 802.2 Header)

IPX I                 Displays the configuration and version of the IPX.COM

program.  Also displays the Hardware Address of the NIC.

NETX I                Displays the version information for NETX.COM

The following commands can be entered at the F:LOGIN> prompt of a DOS

workstation after IPX.COM and NETX.COM have been loaded.

Note: Commands marked with an ╗*╗ may require login.

SLIST                 Displays a list of available Servers and their Network

and Node numbers.  The information for SLIST comes

from the server that the W/S is attached to and is

based on the server's SAP table.  In unusual cases, a

server may appear in the SLIST table and yet the user

may not be able to communicate with the server.

USERLIST /A        *  Displays a list of the users currently connected to

the default server.  The ╗/A╗ option lists the user's

Network and Node numbers also.

WHOAMI             *  Displays the Username, Servername, and Connection

number of the W/S.

FILESERVER COMMANDS: The following commands can be entered at a Novell 3.X

fileserver ╗:╗ prompt.

CONFIG                Displays the basic configuration of the fileserver:

NIC's installed, etc.

LOAD MONITOR          Loads the MONITOR.NLM.  Monitor displays lots of

useful and interesting information, including:

Fileserver Utilization

Connections in Use

NIC Configuration Info and Error Statistics

...and more

MODULES               Lists the Netware Loadable Modules (NLMs) that are

loaded on the server.

TRACK ON              Displays RIP and SAP traffic that is sent or received

by the fileserver.

TRACK OFF             Disables the TRACK ON display

DISPLAY NETWORKS      Displays the router's RIP table including:

IPX Network Numbers

Number of Hops (networks crossed) to reach Network

Estimated Time in ticks (1/18 sec) to reach Network

DISPLAY SERVERS       Displays the router's SAP table including:

Fileservers the router is aware of

Number of Hops (networks crossed) to reach Server

LOAD FILTER.NLM       Loads Novell's SAP Filter.  This NLM allows SAP

broadcasts to be filtered.  Service providers broadcast

SAP packets every 60 seconds.  As the number of servers

increases, the bandwidth used by SAP broadcasts

increases.  FILTER.NLM allows SAP traffic to be reduced.

LOAD LIPX.NLM         Enables Novell's Large Packet Protocol.  Without LIPX,

the maximum packet size that a server will send to

a client on another network is 512 bytes.  With LIPX,

larger packet sizes can be negotiated.

LOAD PBURST.NLM       Enables Novell's Packet Burst Protocol on the server.

Without PBURST, each request must be acknowledged

before the next request can be sent.  This is because

IPX is not a windowed protocol.  PBURST makes IPX into

a sliding window protocol.  With PBURST, up to 64K of

data can be transferred before an acknowledgement is

required.

LOAD ROUTE.NLM        Enables Token Ring Source Routing in the server

RESET ROUTER          Resets the fileserver's RIP Table.  The server normally

updates it's RIP table every two minutes.  The RESET

ROUTER command flushes the RIP table and forces the

server to broadcast a RIP Request to update the table.

VERSION               Displays the fileserver's Version information.