Novell IPX Routing Information Protocol (RIP) format

──────────────────────────────────────────────────────

┌────────────┬─────────────┬─────────────────────────┬─────────────┐

│ MAC Header │ IPX Header  │     RIP Header/Data     │ MAC Trailer │

└────────────┴─────────────┴─────────────────────────┴─────────────┘

│             │                         │

┌────────────┘             └─────────────────┐       └───────────────┐

│                                            │                       │

│                ┌──── Dest ───┬──── Src ────┐ (2)   (4)    (2)   (2)│

┌────┬───┬──┬────┬───┬────┬────┬───┬────┬────┬────┬────────┬────┬────┐

│Cksm│Len│TC│0001│Net│Node│0453│Net│Node│0453│TYPE│NetAddrX│HOPS│TIME│

└────┴───┴──┴────┴───┴────┴────┴───┴────┴────┴────┴────────┴────┴────┘

│             │             │

│             │             └─> SRC SCKT = 0453 (RIP)

│             └───────────────> DST SCKT = 0453 (RIP)

└────────────────────────> IPX PCKT TYPE = 0001 (RIP)

Novell IPX Routing Information Protocol Header Field Descriptions

───────────────────────────────────────────────────────────────────

TYPE:    0001 = General RIP Request

0002 = General RIP Response

NetAddrX:    Network Address

Request Packet = target network for desired routing information.

FFFFFFFF in this field is a request for all known networks.

Response Packet = Up to 50 networks may be advertised per RIP

response packet.

HOPS:    The number of IPX routers that must be crossed in order to

reach NetAddrX.  This field is padded with 0xFFFF for request

packets.   A hop count of 16 indicates an unreachable network.

TIME:    Estimated time to NetAddrX in ╗tics╗.  Each tic = 1/18 seconds.

This field is only valid for reply packets, it is padded with

0xFFFF for request packets.