Novell's Sequenced Packet Exchange (SPX) protocol

───────────────────────────────────────────────────

SPX is a connection oriented, Transport layer protocol.  SPX provides

guaranteed delivery of packets, in the correct sequence.  It provides lost

or out-of-sequence packet detection and correction.

┌────────────┬────────────┬────────────┬────────────┬─────────────┐

│ MAC Header │ IPX Header │ SPX Header │   Data     │ MAC Trailer │

└────────────┴────────────┴────────────┴────────────┴─────────────┘

┌────────────┘            │            │            └─────────────┐

│            ┌────────────┘            └────┐                     │

│    (30)    │(1)(1) (2)  (2)  (2)  (2)  (2)│<─────(variable)────>│

┌────────────┬──┬──┬────┬────┬────┬────┬────┬─────────────────────┐

│ IPX Header │CC│DS│SCID│DCID│SEQ#│ACK#│ALOC│        Data         │

└────────────┴──┴──┴────┴────┴────┴────┴────┴─────────────────────┘

└───────────────────── SPX Length field count ────────────────────┘

CC: Connection Control

0x10 = End of Message (The client wants to end the connection)

0x20 = Attention (Not implemented)

0x40 = Acknowledgement Required (Data sent, ack required)

0x80 = System Packet (Ack packet)

DS: Data Stream Type

0x00 - 0xFD =  Client Defined. Ignored by SPX

0xFE = End of Connection. SPX generates this packet when

a client calls for termination of an active

connection; it is the last message delivered on

the connection.

0xFF = End of Connection Acknowledgement. This is the reply

to the End of Connection request.  SPX generates

this system packet; it is not delivered to the

connected clients.

SCID: Source Connection ID

Another address, similar in function to the IPX Socket.

This number is generated by the source station.

DCID: Destination Connection ID

Number assigned by SPX at the destination.  Connection ID's

allow more than one device to communicate with the same

SPX socket simultaneously.

SEQ#: Sequence Number

A count of packets sent or received.  The sending and

receiving devices keep separate counts.  The number wraps

to 0x0000 after reaching 0xFFFF.

ACK#: Acknowledge Number

The sequence number of the next packet SPX expects to

receive.

ALOC: Allocation Number

The number of listen buffers outstanding in one direction

on the connection.  SPX may only send packets until the

sequence number equals the remote allocation number.

Ethernet / IEEE802.3 / and IEEE802.5 SPX Formats

──────────────────────────────────────────────────

Note: See c:\seminar\files\info\network\ipx\tl100.txt for TL/100 IPX

encapsulation parameter table...

Novell 802.3 ╗Raw╗ Frame Format

─────────────────────────────────

(7)    (1)    (6)       (6)    (2)│<────(46-1500 bytes)────>│(4)

┌────────┬───┬─────────┬─────────┬───┬─────┬─────┬─────────────┬───┐

802.3   │Preamble│SFD│Dest Addr│ Src Addr│Len│ IPX │ SPX │ Data | Pad  │FCS│

└────────┴───┴─────────┴─────────┴───┴─────┴─────┴─────────────┴───┘

┌────────────────────────────────────┘     │     │             │

│            ┌─────────────────────────────┘     │             └───┐

│    (30)    │(1)(1) (2)  (2)  (2)  (2)  (2)┌────┘(4-1458 bytes)   │

┌────────────┬──┬──┬────┬────┬────┬────┬────┬──────────────────────┐

│ IPX Header │CC│DS│SCID│DCID│SEQ#│ACK#│ALOC│         Data         │

└────────────┴──┴──┴────┴────┴────┴────┴────┴──────────────────────┘

└───────────────────── SPX Length field count ─────────────────────┘

Novell Ethernet II Frame Format

─────────────────────────────────

(8)        (6)       (6)    (2) │<────(46-1500 bytes)────>│(4)

┌───────────┬─────────┬─────────┬────┬─────┬─────┬─────────────┬───┐

Eth II  │ Preamble  │Dest Addr│ Src Addr│Type│ IPX │ SPX │    Data     │FCS│

└───────────┴─────────┴─────────┴────┴─────┴─────┴─────────────┴───┘

8137│     │     │             │

┌────────────────────────────────────┘     │     │             │

│            ┌─────────────────────────────┘     │             └───┐

│    (30)    │(1)(1) (2)  (2)  (2)  (2)  (2)┌────┘(4-1458 bytes)   │

┌────────────┬──┬──┬────┬────┬────┬────┬────┬──────────────────────┐

│ IPX Header │CC│DS│SCID│DCID│SEQ#│ACK#│ALOC│        Data          │

└────────────┴──┴──┴────┴────┴────┴────┴────┴──────────────────────┘

└───────────────────── SPX Length field count ─────────────────────┘

Novell 802.3/802.2 Frame Format

─────────────────────────────────

(7)    (1)    (6)       (6)    (2)│<────(46-1500 bytes)────>│(4)

┌────────┬───┬─────────┬─────────┬───┬─────────────────────────┬───┐

802.3   │Preamble│SFD│Dest Addr│ Src Addr│Len│        Data     |  Pad  │FCS│

└────────┴───┴─────────┴─────────┴───┴─────────────────────────┴───┘

┌────────────────────────────────────┘                         └───┐

│ (1)  (1)  (1)  (30)  (12)                                        │

┌────┬────┬────┬─────┬─────┬───────────────────────────────────────┐

802.2   │DSAP│SSAP│CTRL│ IPX │ SPX │            Information                │

└────┴────┴────┴─────┴─────┴───────────────────────────────────────┘

E0   E0   03 │     │     │                                       │

┌──────────────┘     │     │                                       │

│            ┌───────┘     └────────────────┐                      │

│    (30)    │(1)(1) (2)  (2)  (2)  (2)  (2)│     (0-1455 bytes)   │

┌────────────┬──┬──┬────┬────┬────┬────┬────┬──────────────────────┐

│ IPX Header │CC│DS│SCID│DCID│SEQ#│ACK#│ALOC│        Data          │

└────────────┴──┴──┴────┴────┴────┴────┴────┴──────────────────────┘

└───────────────────── SPX Length field count ─────────────────────┘

Novell 802.3/802.2/SNAP Frame Format

──────────────────────────────────────

(7)    (1)    (6)       (6)    (2)│<────(46-1500 bytes)────>│(4)

┌────────┬───┬─────────┬─────────┬───┬─────────────────────────┬───┐

802.3   │Preamble│SFD│Dest Addr│ Src Addr│Len│        Data     |  Pad  │FCS│

└────────┴───┴─────────┴─────────┴───┴─────────────────────────┴───┘

┌────────────────────────────────────┘                         └───┐

│ (1)  (1)  (1)   (5)     (30)  (12)                               │

┌────┬────┬────┬────────┬─────┬─────┬──────────────────────────────┐

802.2   │DSAP│SSAP│CTRL│SNAP Hdr│ IPX │ SPX │        Information           │

└────┴────┴────┴────────┴─────┴─────┴──────────────────────────────┘

AA   AA   03 │        │     │     │                              │

┌─────┘        │     │     │                              │

│   (3)    (2) │     │     │                              │

┌────────┬─────┐     │     │                              │

│00│00│00│Type │     │     │                              │

└────────┴─────┘     │     │                              │

8137 │     │     │                              │

┌───────────────────────┘     │     │                              │

│            ┌────────────────┘     └───────┐                      │

│    (30)    │(1)(1) (2)  (2)  (2)  (2)  (2)│    (0-1450 bytes)    │

┌────────────┬──┬──┬────┬────┬────┬────┬────┬──────────────────────┐

│ IPX Header │CC│DS│SCID│DCID│SEQ#│ACK#│ALOC│        Data          │

└────────────┴──┴──┴────┴────┴────┴────┴────┴──────────────────────┘

└───────────────────── SPX Length field count ─────────────────────┘

Novell 802.5/802.2 Frame Format

─────────────────────────────────

(1)(1)(1)    (6)       (6)   (0-30)│<────(variable)─────>│ (4)(1)(1)

┌──┬──┬──┬─────────┬─────────┬─────┬─────────────────────┬───┬──┬──┐

802.5   │SD│AC│FC│Dest Addr│ Src Addr│ RIF │       Data          │FCS│ED│FS│

└──┴──┴──┴─────────┴─────────┴─────┴─────────────────────┴───┴──┴──┘

┌──────────────────────────────────┘                     └─────────┐

│ (1)  (1)  (1)  (30)  (12)                                        │

┌────┬────┬────┬─────┬─────┬───────────────────────────────────────┐

802.2   │DSAP│SSAP│CTRL│ IPX │ SPX │          Information                  │

└────┴────┴────┴─────┴─────┴───────────────────────────────────────┘

E0   E0   03 │     │     │                                       │

┌──────────────┘     │     │                                       │

│            ┌───────┘     └────────────────┐                      │

│    (30)    │(1)(1) (2)  (2)  (2)  (2)  (2)│ <────(variable)─────>│

┌────────────┬──┬──┬────┬────┬────┬────┬────┬──────────────────────┐

│ IPX Header │CC│DS│SCID│DCID│SEQ#│ACK#│ALOC│        Data          │

└────────────┴──┴──┴────┴────┴────┴────┴────┴──────────────────────┘

└───────────────────── SPX Length field count ─────────────────────┘

Novell 802.5/802.2/SNAP Frame Format

──────────────────────────────────────

(1)(1)(1)    (6)       (6)   (0-30)│<────(variable)─────>│ (4)(1)(1)

┌──┬──┬──┬─────────┬─────────┬─────┬─────────────────────┬───┬──┬──┐

802.5   │SD│AC│FC│Dest Addr│ Src Addr│ RIF │       Data          │FCS│ED│FS│

└──┴──┴──┴─────────┴─────────┴─────┴─────────────────────┴───┴──┴──┘

┌──────────────────────────────────┘                     └─────────┐

│ (1)  (1)  (1)   (5)     (30)  (12)                               │

┌────┬────┬────┬────────┬─────┬─────┬──────────────────────────────┐

802.2   │DSAP│SSAP│CTRL│SNAP Hdr│ IPX │ SPX │        Information           │

└────┴────┴────┴────────┴─────┴─────┴──────────────────────────────┘

AA   AA   03 │        │     │     │                              │

┌─────┘        │     │     │                              │

│   (3)    (2) │     │     │                              │

┌────────┬─────┐     │     │                              │

│00│00│00│Type │     │     │                              │

└────────┴─────┘     │     │                              │

8137 │     │     │                              │

┌───────────────────────┘     │     │                              │

│            ┌────────────────┘     └───────┐                      │

│    (30)    │(1)(1) (2)  (2)  (2)  (2)  (2)│<─────(variable)─────>│

┌────────────┬──┬──┬────┬────┬────┬────┬────┬──────────────────────┐

│ IPX Header │CC│DS│SCID│DCID│SEQ#│ACK#│ALOC│        Data          │

└────────────┴──┴──┴────┴────┴────┴────┴────┴──────────────────────┘

└───────────────────── SPX Length field count ─────────────────────┘