Transmission Control Protocol (TCP) packet format

───────────────────────────────────────────────────

┌────────────┬───────────┬────────────┬──────────────────┬─────────────┐

│ MAC Header │ IP Header │ TCP Header │ Layer 5 Hdr/Data │ MAC Trailer │

└────────────┴───────────┴────────────┴──────────────────┴─────────────┘

│            │

┌─────────────────────┘            └────────────────────────────┐

│0               8              16              24            31│

┌───────────────┼───────────────┼───────────────┴───────────────┐

│         Source Port           │       Destination Port        │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                       Sequence Number                         │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                   Acknowledgement Number                      │

├─┴─┴─┴─┼─┴─┴─┴─┴─┴─┼─┼─┼─┼─┼─┼─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│ Data  │           │U│A│P│R│S│F│                               │

│Offset │ Reserved  │R│C│S│S│Y│I│           Window              │

│       │           │G│K│H│T│N│N│                               │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│          Checksum             │        Urgent Pointer         │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┤

│                 Options                       │   Padding     │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│                              Data                             │

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘

Source Port: Source TCP port address.  This address identifies a

process or service that resides at the source host.

Destination Port: Destination TCP port address.  This address identifies

a process or service that resides at the target host.

Sequence Number: Sequence number of the first data byte in the segment.

If SYN is set, then the sequence number is the

Initial Sequence Number (ISN) of the connection.

Acknowledgement Number: If the ACK bit is set, this contains the value of the

next sequence number that the sender expects to

receive.

Data Offset: Number of 32-bit words in the TCP header (range:5-15)

Control Flags: Single bit indicators used to establish, maintain,

and terminate connections.

URG: Urgent Pointer field significant (URG=1)

ACK: Acknowledgement field significant (ACK=1)

PSH: Push Function (PSH=1).  Tells receiving TCP

to immediately deliver the data in this segment

to the upper layer protocol.

RST: Resets the connection in the case of duplicate

SYN's, host crashes, etc.  (RST=1)

SYN: Used to establish connections (SYN=1).

FIN: Used to terminate connections (FIN=1).

Window: The number of bytes, beginning with the one indicated

in the Acknowledgement field, which the sender is

willing to accept.

Checksum: Applied to the TCP header and data

Urgent Pointer: Specifies the last byte of Urgent data (URG=1)

Options:

End Of Options List: Indicates the end of the options

list.

No Operation: May be used between options to align

the beginning of a subsequent option on a 32-bit

boundary.

Maximum Segment Size: Indicates the maximum receive

segment size at the TCP which sends the segment.

Must only be seent in the initial connection request.