User Datagram Protocol Packet Format

──────────────────────────────────────

┌────────────┬───────────┬────────────┬──────────────────┬─────────────┐

│ MAC Header │ IP Header │ UDP Header │ Layer 5 Hdr/Data │ MAC Trailer │

└────────────┴───────────┴────────────┴──────────────────┴─────────────┘

│            │

┌─────────────────────┘            └────────────────────────────┐

│0               8              16              24            31│

┌───────────────┼───────────────┼───────────────┴───────────────┐

│         Source Port           │       Destination Port        │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┼─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

│            Length             │          Checksum             │

├─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┤

|                         Layer 5 data....                      |

└─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┴─┘

Source Port: Source UDP port in the sending host.  An address that

identifies a process or service in the sending host.

The address also specifies the port to which replies

should be sent.

Destination Port: Destination UDP port in the target host.  An address that

identifies a process or service in the target host.

Length: Number of bytes in the UDP header including user data.

Checksum: Applies to all bytes in the UDP header and data.