Name: ....................................

1.) What is the principle difference between IPX RIP and NLSP ?

2.) Which of the following is true about a NLSP pseudonode ?

( ) it is the virtual mac-address used when configuring a router

( ) it is the client not attached yet to the server

( ) it is the so-called "seed-server"

( ) on LAN links a pseudonode is represented by the DR

3.) Does NLSP use hellos ?

( ) sometimes

( ) yes, always

( ) only when command "ipx keepalive enable" was used

( ) never

4.) Which of the following is true about a NLSP Area ID ?

( ) unique ID, coming from Novell, like an autonomous system

( ) to be used when IPXWAN is configured

( ) used when redistributing between RIP and NLSP

( ) only used when doing Level 1-Intradomain-routing

5.) NLSP metrics are  _____  ?   ticks, hops, cost ?

( ) ticks and hops

( ) ticks and cost

( ) just cost

6.) Describe briefly NLSP Priority Escalation:

7.) NLSP's routing algorithm is also known as:

( ) garcia-luna-algorithm

( ) dijkstra algorithm

( ) bellman-ford-algorithm

( ) jitter-algorithm

8.) Describe the NLSP jitter algorithm ?

9.) Whats true about the NLSP hello timers ?

( ) defaults to 20 seconds

( ) defaults to 10 seconds

( ) defaults are: 20 sec for non-DR's and 10 seconds for DR's

10.) Whats true about NLSP-RIP/SAP-redistribution ?

( ) it works automatic as long as all servers are in the same

area and level 2-routing is configured

( ) must be configured manually

( ) works automatic, just like that

11.) What's the NLSP MAX AGE value ?

( ) 10 sec

( ) 30 sec

( ) 60 sec

( ) 7200 sec

12.) Are there any metric differences with NLSP between

Ethernet and a 16 Mb-Token Ring ?

( ) no differences

( ) yes, ethernet 10, token ring 15

( ) yes, ethernet 20, token ring 19

( ) yes, ethernet 17, token ring 16

13.) What does "IPX floating static route" mean ?

( ) a new feature planned for future IOS-releases

( ) allows static routes to be overridden by dynamically learned

route

( ) when dynamic routing fails for any reason, the preconfigured

static route takes presedence

14.) What's the default priority of a NLSP-router ?

( ) 10

( ) 100

( ) 44

15.) What disables fast switching in IPX ?

( ) no decnet switching

( ) no ipx fast-switching

( ) no ipx route cache

16.) NLSP is:

( ) derived from IS-IS (with a special Novell header)

( ) derived from XNS with changes from dec to hex

( ) not derived from anything or anyone

17.) DSAP and SSAP value of 0xE0. It is:

( ) a so-called "novell raw"-frame

( ) a novell 3.x default frame

( ) a novell 802.2 frame

18.) Let's say you have to troubleshoot NLSP and you would like

to view the LSPs from a novell-server ?

( ) this is not possible, you must debug it at a router

( ) you must use TRACK ON

( ) use an undocumented function when loading IPXCON:

load IPXCON /P

19.) Which command gives information about programs that are

running in the server ?

( ) modules

( ) sh prot

( ) load install

( ) syscon

20.) Which of the following is true about NLSP LSPs ?

( ) they are always sequenced and acknowledged

( ) not acknowledged on LANs, acknowledged on WANs only

( ) only acknowledged between servers and routers

( ) LSP are never acknowledged

21.) What's the default IPX-encapsulation on a Cisco

Ethernet-interface in Novell-syntax:

( ) novell-ether

( ) Ethernet_802.2

( ) Ether_SNAP

( ) Ethernet_802.3

22.) What`s the NLSP router dead value ?

( ) 3 times the hello-timer

( ) 3 times 30 seconds

( ) 180 seconds

23.) Does NLSP support load balancing ?

( ) up to 8 equal-cost paths

( ) up to 64 paths

( ) up to 30 paths

( ) no load balancing with NLSP

24.) Does NLSP use multicasts ?

( ) no, but numbered and sequenced LSPs

( ) yes, always

( ) yes and no, it's server-configurable, default: yes

( ) no, it's save by other means

25.) Which of the following is true about "router dead interval"?

( ) waiting time until the BDR takes over the kingdom

( ) waiting time until neigbor router is marked unreachable

( ) waiting time until the SPF algorithm is executed

26.) How do you control who's DR ?

27.) What's NLSP's default packet size ?

( ) 265 bytes

( ) 512 bytes

( ) 1024 bytes

28.) What`s the advantage of NLSP over RIP ?