Name: ....................................             \test4.doc

1.) How could you look at the OSPF timer settings, if

defaults are used ?

( ) SH IP OSPF process-id

( ) SH IP OSPF TIMERS

( ) SH IP TIMERS

2.) You are running IOS 11.1(8) or higher. What does the

command "u all"  do ?

( ) sorry, I don't know this command

( ) command does not exist

( ) UPDATE ALL CDP NEIGHBORS

( ) UNERASE ALL (configs in NVRAM)

( ) UNDEBUG ALL

3.) What's the default privilege level ?

( ) 1

( ) 15

( ) 10

( ) none of above

4.) What's the defaults for the AUX-port  ?

( ) 9600 baudrate, no parity, 2 stopbits, 8 databits

( ) 9600 baudrate, no parity, 1 stopbit, 8 databits

( ) 9600 baudrate, even parity, 2 stopbits, 8 databits

( ) 9600 baudrate, even parity, 1 stopbit, 8 databits

5.) To remove all routing table entries, you use: ?

( ) flush route cache

( ) clear route cache

( ) clear ip route *

( ) clear ip ro all

6.) To clear the fast-switching cache , you do: ?

( ) clear fast cashe

( ) clear switch cache

( ) clear arp cache

( ) clear all cache

7.) What are legal values if using the "distance" router-

configuration command ?

( ) integer from 0 - 255

( ) integer from 1 - 255

( ) integer from 10 - 255

( ) integer from 10 - 254

8.) How do you express 100 % traffic loading when setting

metrics for IGRP or EIGRP ?

( ) 0   (means 100% loading)

( ) 255 (means 100% loading)

( ) 100 (means 100% loading)

( ) 1   (means 100% loading)

9.) When setting the Route Delay for IGRP or EIGRP, the

value is expressed in ?

( ) microseconds

( ) tens of microseconds

( ) any positive or negative number that is a multiple

of 39.1 nanoseconds

( ) ticks

10.) IP routing tables show metrics and admin-distances in

brackets when you do SH IP RO. Which is shown first ?

( ) the metric value

( ) the administrative distance

( ) the composite metric

11.) X.25 configuration: You must configure a switch.

Which is the correct statement ?

( )  encaps X25

( )  encaps X25 switch

( )  encaps X25 dce

( )  none of above

12.) Which of the following is a correct Netware network ?

( ) DEADBEEF9

( ) C0FFEE

( ) FooFoo

13.) In IPX routing tables you might see a lower-case "c"

in the first column. What does it mean ?

( ) a directly connected network, what else?

( ) a candidate default network

( ) directly connected primary network

( ) directly connected secondary net

14.) DECNet MOP System-ID-packets from DECLANBridge are

seen every ______ ?

( ) 15 seconds

( ) 40 seconds

( ) 8-10 minutes

15.) DECNet Phase IV endnode Hello packets are sent by each

DECNet host every _______ ?

( ) 15 seconds

( ) 40 seconds

( ) 60 seconds

( ) never

16.) To configure the router discovery feature using the

Cisco Gateway Discovery Protocol, use which command ?

( ) IP GDP GDP

( ) IP GDP

( ) IP GDP CISCO

( ) none of above

17.) What's a Supernet ?

18.) What does CLEAR IP ROUTE * do ?

19.) Ethertype-field:  DECNet Phase IV Router Hello Packets

use which protocol type ?

( ) 6001

( ) 6002

( ) 6003

20.) To define a named IP access list, you use the following

global configuration command ?

( ) ACCESS-LIST name + conditions

( ) ACCESS-LIST STANDARD name + conditions or

ACCESS-LIST EXTENDED name + conditions

( ) IP ACCESS-LIST STANDARD name or

IP ACCESS-LIST EXTENDED name

( ) none of above

21.) What's the maximum of name servers you can specify

in the IP NAME-SERVER command ?

( ) 6 name-servers

( ) 9 name-servers

( ) 128 name-servers

( ) up to 255 name-servers

22.) What`s the purpose of the hop counts in an IGRP packet ?

23.) The IP NETMASK-FORMAT command is a ......  ?

( ) global configuration command

( ) line configuration command

( ) interface configuration command

( ) router configuration command

( ) sub-interface configuration command

24.) Which IPX datagram field provides a similar function to

avoid a loop as in the TTL-field in IP datagrams ?

( ) same TTL-field as in IP

( ) Poison Reverse field

( ) Transport Control field

( ) Maximum Packet Lifetime (MPL) field

( ) Hop-count field

25.) What's Ethernet Protocol type 6004 ?

( ) DEC MOP Dump/Load Assistance

( ) DEC unassigned

( ) DEC LAT

26.) What does the CLEAR ARP-CACHE command do ?

( ) deletes all dynamic entries from the ARP cache

( ) deletes all dynamic entried from the ARP cache and

cleares the fast-switching cache

( ) deletes all dynamic entries from the ARP cache and

clears the IP route cache

27.) Which IPX datagram field points to the next higher

layer, as the Protocol field in IP ?

( ) Packet Type

( ) Protocol Type

( ) SPP-field

( ) Cicuit Type

( ) Version field

( ) NCP field