Name: ....................................             \test5.doc

1.) What does the "BRIDGE nnn ADDRESS xxxx.xxxx.xxxx DISCARD" do ?

( ) source macaddress xxxx is denied to be bridged

( ) destination macaddress xxx is denied to be bridged

( ) all broadcasts are to be forwarded

2.) There are 8 different "trap-levels" (logging levels) . Which is the

default ?

( ) 0 , emergencies

( ) 1 , alerts

( ) 4 , warnings

( ) 6 , informational

3.) Given is a DDP-datagram with a source-socket 1 . What is it ?

( ) a NBP request

( ) a RTMP update

( ) a ZIP reply

( ) an ATP acknowledgement

4.) You do a "SH IPX RO" and you see a "RH" in the left column of an entry.

What does it mean ?

( ) novell address gleaning is enabled

( ) it's a novell host-explicit route

( ) holddown-timer for this entry is configured to zero

( ) a novell route in holddown

5.) A DSAP/SSAP value of "42" means ?

( ) ARAP

( ) AAA

( ) DDLU

( ) BPDU

6.) A DDP datagram with a source-socket of "6" is ?

( ) a NBP request

( ) a SAP reply

( ) a ZIP query

( ) a RTMP update

7.) How much room has the "LOGGING BUFFERED" command to store output ? ?

( ) as much as RAM last

( ) exactly 32 KB RAM

( ) unlimited

( ) about 100 lines of output in a wrap-around-buffer

8.) What does the "BRIDGE-GROUP nnn OUTPUT-TYPE-LIST mmm"  do ?

( ) filtering based on protocol

( ) filtering based on BPDU packets

( ) filtering based on Spanning Tree (DEC or IEEE)

( ) filtering based on applications

9.) ACLs 700-799 show macaddresses (with wildcards). Which of the following

is true ?

( ) addresses shown are always source-addresses

( ) addresses shown are always destination-addresses

( ) addresses shown are always source- and destination-addresses

( ) addresses are either sources, or destinations, or both, depending

on how the access-lists are enabled

10.) A DDP datagram with a source-socket of "2" is ?

( ) a NBP packet

( ) a SAP reply

( ) an AFP packet

( ) a RTMP update

11.) AppleTalk is using which frame encapsulation ?

( )  ethertype

( )  802.3 with a proprietary header following

( )  SNAP

( )  ethertype with a 802.2 LLC header

12.) What's a NCB ?

13.) The header of a RTMP packet contains: ?

( ) the router's network number, ID length, router's node-ID

( ) number of rtmp tuples

( ) RTMP version, networks, hops

( ) transaction-ID and sequence number

14.) What's the maximum size for a TR ring number ?

( ) 10 bits

( ) 12 bits

( ) 16 bits

15.) OSPF routes that originate from different OSPF processes are

called: ?

( ) intra-domain routes

( ) external routes

( ) inter-domain routes

( ) stubby routes

16.) What's the OSPF router-ID, if you have several loopback-interfaces

with IP addresses ?

( ) the first loopback-address found in sequential order

( ) the highest loopback ip address

( ) the last loopback-address

( ) random

17.) What's the OSPF router dead timer ?

18.) The default for the interface OSPF priority is ?

( )  1

( )  100

( )  10

19.) What does the interface state DROTHER mean ?

20.) ?

( ) ACCESS-LIST name + conditions

( ) ACCESS-LIST STANDARD name + conditions or

ACCESS-LIST EXTENDED name + conditions

( ) IP ACCESS-LIST STANDARD name or

IP ACCESS-LIST EXTENDED name

( ) none of above

21.) What's the maximum of name servers you can specify

in the IP NAME-SERVER command ?

( ) 6 name-servers

( ) 9 name-servers

( ) 128 name-servers

( ) up to 255 name-servers

22.) What`s the purpose of the hop counts in an IGRP packet ?

23.) The IP NETMASK-FORMAT command is a ......  ?

( ) global configuration command

( ) line configuration command

( ) interface configuration command

( ) router configuration command

( ) sub-interface configuration command

24.) Which IPX datagram field provides a similar function to

avoid a loop as in the TTL-field in IP datagrams ?

( ) same TTL-field as in IP

( ) Poison Reverse field

( ) Transport Control field

( ) Maximum Packet Lifetime (MPL) field

( ) Hop-count field

25.) What's Ethernet Protocol type 6004 ?

( ) DEC MOP Dump/Load Assistance

( ) DEC unassigned

( ) DEC LAT

26.) What does the CLEAR ARP-CACHE command do ?

( ) deletes all dynamic entries from the ARP cache

( ) deletes all dynamic entried from the ARP cache and

cleares the fast-switching cache

( ) deletes all dynamic entries from the ARP cache and

clears the IP route cache

27.) Which IPX datagram field points to the next higher

layer, as the Protocol field in IP ?

( ) Packet Type

( ) Protocol Type

( ) SPP-field

( ) Cicuit Type

( ) Version field

( ) NCP field